3GPP TSG RAN WG3 Meeting #100	Tdoc R3-183209

Busan, Korea, 21st – 25th May

Agenda Item:	23.2

Source:	Ericsson

Title:	E1 multiple SCTP associations - for 38.462

Document for:	pCR TS 38.462

# Introduction

In R3-183210 we re-iterate on the need for supporting multiple SCTP associations over E1 for enabling user-plane virtualization. In this contribution, we propose the required changes to TS 38.462 to support multiple SCTP associations on E1. Note that the changes aim at alignment with TS 38.412 (NG) and TS 38.472 (F1).

RAN3 is kindly asked to agree with TP for TS 38.463 in Annex I.

# Annex I: pCR for TS 38.462

<<<<<< START CHANGE >>>>>>

SCTP (IETF RFC 4960 [6]) shall be supported as the transport layer of E1 signalling bearer. The Payload Protocol Identifier assigned by IANA to be used by SCTP for the application layer protocol E1AP is xxxx (FFS).

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

gNB-CU-CP and gNB-CU-UP shall support a configuration with a single SCTP association per gNB-CU-CP / gNB-CU-UP pair. Configurations with multiple SCTP endpoints per gNB-CU-CP / gNB-CU-UP pair should be supported. When configurations with multiple SCTP associations are supported, the gNB-CU-CP may request to dynamically add/remove SCTP associations between the gNB-CU-CP / gNB-CU-UP pair. Within the set of SCTP associations established between one gNB-CU-CP and gNB-CU-UP pair, a single SCTP association shall be employed for E1AP elementary procedures that utilize non-UE-associated signalling with the possibility of fail-over to a new association to enable robustness.

Either the gNB-CU-CP or gNB-CU-UP shall establish the SCTP association. The SCTP Destination Port number value assigned by IANA to be used for E1AP is yyyyy (FFS).

Between one gNB-CU-CP and gNB-CU-UP pair:

-	a single pair of stream identifiers shall be reserved over an SCTP association for the sole use of E1AP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers over one or several SCTP associations shall be reserved for the sole use of E1AP elementary procedures that utilize UE-associated signalling. However, a few pairs (i.e. more than one) should be reserved.

-	For a single UE-associated signalling the gNB-CU-CP and the gNB-CU-UP shall use one SCTP association and one SCTP stream, and the stream should not be changed during the communication of the UE-associated signalling unless TNL binding update is performed.

Transport network redundancy may be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy an INIT may be sent from gNB-CU-CP or gNB-CU-UP, at any time for an already established SCTP association, which shall be handled as defined in IETF RFC 4960 [6] in sub clause 5.2.

The SCTP congestion control may, using an implementation specific mechanism, initiate higher layer protocols to reduce the signalling traffic at the source and prioritise certain messages.

<<<<<< END CHANGE >>>>>>