3GPP TSG RAN WG3 #100	Tdoc R3-183216

Busan, Korea, 21st – 25th May

Agenda Item:	23.5

Source:	Ericsson

Title:	Rapporteur update to TS 38.463

Document for:	Discussion

# Introduction

This document presents the Rapporteur update to TS 38.463.

# Discussion

- The main objective of this Rapporteur update is to introduce a basic structure for the E1AP ASN.1. The ASN.1 only contains that Error Indication and Reset procedures. More procedures will be added in the coming weeks following the structure presented in this Rapporteur update.

- Some Typos have been corrected throughout the spec.

- The optionality of S-NSSAI is change from optional to mandatory everywhere, following agreement that slicing is mandatory feature.

- In the Bearer Context Modification Request, PDU Session Resource to Modify List, DRB to Setup List: optionality of SDAP-Configuration and PDCP-Configuration is changed from optional to mandatory to align with the other cases of DRB setup where this information is mandatory.