3GPP TSG RAN WG3 Meeting #100	Tdoc R3-183306

Busan, Korea, 21st – 25th May

Agenda Item:	31.3.1.3

Source:	Ericsson

Title:	Response to R3-182862

Document for:	TP for BL SA CR for TS 38.473

# Introduction

R3-182862 addresses the issue of how the gNB-CU can inform the gNB-DU that the UE is going to be released before F1AP UE Context Setup procedure is performed (e.g., right after Initial UL RRC Message Transfer). The authors propose to introduce a new UE Reject Indication IE in the DL RRC Message Transfer.

First, we thank the authors of R3-182862 for raising this important issue. Then, in this contribution we provide our view on the subject.

# Discussion

As explained in R3-182862, there are several cases where the gNB-CU may need to release the UE without performing UE Context Setup procedure. Two examples are:

- Admission failure at the gNB-CU: after receiving the Initial UL RRC Message Transfer, the gNB-CU cannot accept the UE (e.g., due to overload) and decides to reject it.

- Signalling only connection: the gNB-CU may decide to establish a signalling only connection using only SRB1. In this case, the gNB-CU does not need to perform a F1AP UE Context Setup procedure. After the signalling between the UE and the network is terminated, the gNB-CU may decide to release the RRC connection by sending the RRC-Release message to the UE.

The authors propose to introduce a new UE Reject Indication IE in the DL RRC Message Transfer to enable the gNB-CU to inform the gNB-DU that the UE is going to be released. Indeed, it may be beneficial to introduce such indication. However, we see the following two problems:

- Additional complexity added in the gNB-DU for processing the DL RRC Message Transfer: every time that the gNB-DU receives a DL RRC Message Transfer it needs to check for the presence of this optional IE. This introduces extra-complexity in the gNB-DU and increases the latency for delivering the RRC messages to the UE. We believe that optional information in the DL RRC Message Transfer should be avoided unless strictly necessary.

- Unclear interaction with the UE Context Release procedure: the proposed solution raises the issues of what the gNB-DU should do when receiving the UE Reject Indication IE in the DL RRC Message Transfer after the F1AP UE Context Setup procedure has been performed. Should the gNB-DU ignore this indication or should the gNB-DU release the UE Context? If the latter, then the UE Context Release procedure becomes redundant. In general, we prefer that each procedure and IE have a single and well-defined scope.

Based on these considerations, we believe that introducing a UE Reject Indication IE in the DL RRC Message Transfer is not a good approach. An alternative solution, which would avoid both the problems mentioned above, is to introduce a new procedure for informing the gNB-DU that the UE is going to be released.

In this solution, the gNB-CU sends a new message F1AP “UE Signalling Connection Release Command” to the gNB-DU. The message contains the RRC-Release/Reject message for the UE. Then, the gNB-DU responds with a new response message F1AP “UE Signalling Connection Release Complete” to indicate the release of all the resources allocated for this UE.

Proposal 1	Introduce a new class-1 procedure to release a signalling connection over F1 (to be used before a UE context setup procedure is performed),

Proposal 2	RAN3 is kindly asked to agree with the TP in Annex I.

# Conclusion

In this contribution, we discussed how to inform the gNB-DU that a UE is going to be released.

Proposal 1	Always rely on the F1AP UE Context Release procedure to inform the gNB-DU that the UE is going to be released.

Proposal 2	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for TS 38.473

<<<<<<<<<<<<<<		FIRST CHANGE		>>>>>>>>>>>>>>>

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM |  |
| UE Signalling Connection Release | UE SIGNALLING CONNECTION RELEASE COMMAND | UE SIGNALLING CONNECTION RELEASE COMPLETE |  |

<<<<<<<<<<<<<<		SECOND CHANGE	>>>>>>>>>>>>>>>

### 8.3.X	UE Signalling Connection Release

#### 8.3.X.1	General

The purpose of the UE Signalling Connection Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection before the gNB-CU has requested the creation of a UE context in the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: UE Signalling Connection Release procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE SIGNALLING CONNECTION RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE SIGNALLING CONNECTION RELEASE COMMAND message, the gNB-DU shall release all related signalling resources for the UE and reply with the UE SIGNALLING CONNECTION RELEASE COMPLETE message.

<<<<<<<<<<<<<<		THIRD CHANGE	>>>>>>>>>>>>>>>

#### 9.2.2.X	UE SIGNALLING CONNECTION RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection for a UE for which the gNB-CU has not requested the creation of a UE context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | M |  | 9.3.1.6 |  | YES | Ignore |

#### 9.2.2.X	UE SIGNALLING CONNECTION RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connection.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |