3GPP TSG-RAN WG3 Meeting #100	R3-183410

# Busan, South Korea, 21st- 25th May  2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Correction on Additional QoS information

Document for:	pCR

# 1	Discussion

The Additional QoS information is introduced in RAN3#99bis meeting.

However it would not be reasonable to indicate that t GBR QoS flow would be the most likely traffic in the PDU session, as the GBR QoS flows are meant to be fulfilled.

Our thinking would be only when the QoS flow is non-GBR characteristic, it might be given as the most likely traffic for the PDU session.

Thus we propose to limit the IE only apply to Non-GBR.

Proposal 1: RAN3 to agree to make the Additional QoS information conditional, i.e. only apply to non GBR QoS flows.

# Proposals

Proposal 1: RAN3 to agree to make the Additional QoS information conditional, i.e. only apply to non GBR QoS flows.

# Text Proposal for NGAP TS 38.413 v0.8.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.12	QoS Flow Level QoS Parameters

Editor’s Note:	Further details FFS.

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  |
| >Non-dynamic 5QI |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  |
| >Dynamic 5QI |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS Flows only. |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of non-GBR QoS flows and shall be ignored otherwise. |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of non-GBR QoS flows and shall be ignored otherwise. |
| PPI | O |  | INTEGER (1..8, …) | Paging Policy Indicator used in PPD (Paging Policy Differentiation). See details in TS 23.501 [9]. This IE applies to PDU sessions of IP type. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>