3GPP TSG-RAN WG3 #100  	R3-183424

# Busan, South Korea, 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0043 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on data existence indication for split bearer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO,INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-5-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Scheduling delay may happen on split bearer when the amount of data is small. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define indication from the node hosting PDCP to corresponding node i.e. “data exists” and “No data exists” for the bearer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Avoiding scheduling delay would be difficult. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 5.5.2.1, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----Unchanged sections are omitted---

5.4.2	Downlink Data Delivery Status

5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer. The corresponding node may also transfer uplink user data for the concerned data radio bearer to the node hosting the NR PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the successful delivery of DL control data to the corresponding node.

When the corresponding node decides to trigger the Feedback for Downlink Data Delivery procedure it shall report as specified in section 5.2:

a)	in case of RLC AM, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity;

b)	the desired buffer size in bytes for the concerned data radio bearer;

c)	the minimum desired buffer size in bytes associated with all data radio beares configured for the UE;

d)	the NR-U packets that were declared as being "lost" by the corresponding node and have not yet been reported to the node hosting the NR PDCP entity within the DL DATA DELIVERY STATUS frame;

e)	if retransmission NR PDCP PDUs have been delivered, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those retransmission NR PDCP PDUs received from the node hosting the NR PDCP entity;

f)	if retransmission NR PDCP PDUs have been transmitted, the highest NR PDCP PDU sequence number transmitted to the lower layers among those retransmission NR PDCP PDUs received from the node hosting the NR PDCP entity;

g)	the highest NR PDCP PDU sequence number transmitted to the lower layers among those NR PDCP PDUs received from the node hosting the NR PDCP entity.

NOTE1:	If a deployment has decided not to use the Transfer of Downlink User Data procedure, f) above is not applicable.

NOTE2:	If DRX is configured to the UE in the corresponding node, b) and c) may be set to "0" during OFF period.

----Unchanged sections are omitted---

5.5.2	Frame format for the NR user plane protocol

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | Spare bit extended flag |  | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  |  |  | Buffered NR PDCP PDU Status flag | Buffered NR PDCP PDU Status Ind |  |  | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

----Unchanged sections are omitted---

5.5.3	Coding of information elements in frames

----Unchanged sections are omitted---

5.5.3.X	Buffered NR PDCP PDU Status Ind

Description: This parameter indicates the presence of Buffered NR PDCP PDU Status.

Value range: {0= Buffered NR PDCP PDU Status not present, 1= Buffered NR PDCP PDU Status present}.

Field length: 1 bit.

5.5.3.X+1	Buffered NR PDCP PDU Status flag

Description: This parameter indicates whether the node hosting PDCP has some buffered NR PDCP PDUs for the concerned data bearer.

Value range: {0= the node hosting PDCP has no more buffered NR PDCP PDUs for the concerned radio bearer, 1= the node hosting PDCP has some buffered NR PDCP PDUs for the concerned radio bearer}.

Field length: 1 bit.

----Unchanged sections are omitted---