3GPP TSG-RAN WG3#101	R3-184411

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	31.3.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.413): Correction of Release inactive to idle in 38.413

Document for:	Discussion and Decision

# 1	Description and proposal

It was agreed at last RAN3#NR AH1807 that when the NG-RAN node receives a UE Context Release Command in inactive state, it will reply with UE Context Release Complete. As described in section 4.2.6 of TS23.502, the User Location Information IE (ULI) is then reported in UE Context Release Complete message. This is already covered by RAN3 stage 3 in TS 38.413.

However, SA2 has further decided in [3] that ULI should be reported in UE Context Release Complete message together with an age (timestamp) for the inactive state. See TS 23.501:

User Location Information is already part of UE CONTEXT RELEASE COMPLETE in 38.413 but not the Age of Location Information (needed in case of RRC-INACTIVE);

In case of inactive state this should include the age of location of the last known cell when current cell is not known. This is needed e.g. when the UE Context Release Complete is sent without/before paging the UE for release.

It should be noticed that this requirement has already been covered in our TS 38.413 for the Location Report case and is currently an optional IE in the Location Report message called “Timestamp”.

There are therefore two options to satisfy SA2 requirement:

- Either we include the Timestamp IE also in the UE Context Release Complete message as an optional IE,

- Or we can factorize this Timestamp IE as an optional attribute of the ULI (i.e. ULI sent sometimes with timestamp, sometimes without).

We slightly prefer the second approach as cleaner.

Proposal: add the Timestamp IE as an optional IE in the User Location Information IE to be also reported in the UE Context Release Complete message.

# 2	References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- S2-187416, User Location Information added in N2 UE Context Release Complete, CR0532 TS 23.502

# Text Proposal for TS 38.413

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | OCTET STRING | This IE may need to be refined | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.11.3	LOCATION REPORT

This message is used to provide the UE's location.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | ignore |
| UE Presence in Area of Interest List | O |  | 9.3.1.67 |  | YES | ignore |
| Location Reporting Request Type | M |  | 9.3.1.65 | Contains the Location Reporting Request Type to which the Location Report refers. | YES | ignore |

Next Change

#### 9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  |
| >E-UTRA user location information |  |  |  |  |
| >>TAI | M |  | 9.3.3.11 |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >>Age of location | O |  | Timestamp 9.3.1.75 | This IE may be included for RRC_inactive state when the current E-UTRA CGI is unknown. |
| >NR user location information |  |  |  |  |
| >>TAI | M |  | 9.3.3.11 |  |
| >>NR CGI | M |  | 9.3.1.7 |  |
| >>Age of location | O |  | Timestamp 9.3.1.75 | This IE may be included for RRC_inactive state when the current NR CGI is unknown. |
| >N3IWF user location information |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. |

Next Change

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformation,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverCommandTransfer,

HandoverPreparationUnsuccessfulTransfer,

HandoverRequiredTransfer,

HandoverRequestAcknowledgeTransfer,

HandoverResourceAllocationUnsuccessfulTransfer,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NewSecurityContextInd,

NGRAN-CGI,

NGRANTraceID,

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

PagingDRX,

PagingOrigin,

PagingPriority,

PathSwitchRequestAcknowledgeTransfer,

PathSwitchRequestSetupFailedTransfer,

PathSwitchRequestTransfer,

PathSwitchRequestUnsuccessfulTransfer,

PDUSessionID,

PDUSessionResourceModifyConfirmTransfer,

PDUSessionResourceModifyIndicationTransfer,

PDUSessionResourceModifyRequestTransfer,

PDUSessionResourceModifyResponseTransfer,

PDUSessionResourceModifyUnsuccessfulTransfer,

PDUSessionResourceNotifyTransfer,

PDUSessionResourceNotifyReleasedTransfer,

PDUSessionResourceReleaseCommandTransfer,

PDUSessionResourceSetupRequestTransfer,

PDUSessionResourceSetupResponseTransfer,

PDUSessionResourceSetupUnsuccessfulTransfer,

PLMNSupportList,

PWSFailedCellIDList,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RAN-UE-NGAP-ID,

RelativeAMFCapacity,

RepetitionPeriod,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SupportedTAList,

TAI,

TAIListForRestart,

TargetID,

TargetToSource-TransparentContainer,

TimeToWait,

TNLAssociationList,

TNLAssociationUsage,

TNLAssociationWeightFactor,

TraceActivation,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-ConnectionItem,

UEContextRequest,

UEIdentityIndexValue,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UESecurityCapabilities,

UnavailableGUAMIList,

UserLocationInformation,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupItem,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddItem,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveItem,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateItem,

id-AMF-TNLAssociationToUpdateList,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformation,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverType,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-LocationReportingRequestType,

id-MaskedIMEISV,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-NAS-PDU,

id-NASC,

id-NewAMF-UE-NGAP-ID,

id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRANTraceID,

id-NR-CGI,

id-NRPPa-PDU,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-PagingDRX,

id-PagingOrigin,

id-PagingPriority,

id-PDUSessionResourceAdmittedItem,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyItemModRes,

id-PDUSessionResourceFailedToModifyListModRes,

id-PDUSessionResourceFailedToSetupItemCxtRes,

id-PDUSessionResourceFailedToSetupItemHOAck,

id-PDUSessionResourceFailedToSetupItemPSReq,

id-PDUSessionResourceFailedToSetupItemSURes,

id-PDUSessionResourceFailedToSetupListCxtRes,

id-PDUSessionResourceFailedToSetupListHOAck,

id-PDUSessionResourceFailedToSetupListPSReq,

id-PDUSessionResourceFailedToSetupListSURes,

id-PDUSessionResourceHandoverItem,

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceItemHORqd,

id-PDUSessionResourceListHORqd,

id-PDUSessionResourceModifyItemModCfm,

id-PDUSessionResourceModifyItemModInd,

id-PDUSessionResourceModifyItemModReq,

id-PDUSessionResourceModifyItemModRes,

id-PDUSessionResourceModifyListModCfm,

id-PDUSessionResourceModifyListModInd,

id-PDUSessionResourceModifyListModReq,

id-PDUSessionResourceModifyListModRes,

id-PDUSessionResourceNotifyItem,

id-PDUSessionResourceNotifyList,

id-PDUSessionResourceReleasedItemNot,

id-PDUSessionResourceReleasedItemPSAck,

id-PDUSessionResourceReleasedListNot,

id-PDUSessionResourceReleasedListPSAck,

id-PDUSessionResourceSetupItemCxtReq,

id-PDUSessionResourceSetupItemCxtRes,

id-PDUSessionResourceSetupItemHOReq,

id-PDUSessionResourceSetupItemSUReq,

id-PDUSessionResourceSetupItemSURes,

id-PDUSessionResourceSetupListCxtReq,

id-PDUSessionResourceSetupListCxtRes,

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSwitchedItem,

id-PDUSessionResourceSwitchedList,

id-PDUSessionResourceToBeSwitchedDLItem,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToReleaseItemHOCmd,

id-PDUSessionResourceToReleaseItemRelCmd,

id-PDUSessionResourceToReleaseListHOCmd,

id-PDUSessionResourceToReleaseListRelCmd,

id-PLMNSupportList,

id-PWSFailedCellIDList,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RAN-UE-NGAP-ID,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SupportedTAList,

id-TAI,

id-TAIItem,

id-TAIList,

id-TAIListForRestart,

id-TargetID,

id-TargetToSource-TransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-ConnectionItem,

id-UE-associatedLogicalNG-ConnectionListResAck,

id-UEContextRequest,

id-UEIdentityIndexValue,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UESecurityCapabilities,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

maxnoofNGConnectionsToReset,

maxnoofTAIs,

maxnoofTNLAssociations,

maxnoofPDUSessions

FROM NGAP-Constants;

Next Change

-- **************************************************************

--

##### -- LOCATION REPORT

--

-- **************************************************************

LocationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportIEs} },

...

}

LocationReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-UserLocationInformation				CRITICALITY ignore	TYPE UserLocationInformation				PRESENCE mandatory	}|

{ ID id-UEPresenceInAreaOfInterestList		CRITICALITY ignore	TYPE UEPresenceInAreaOfInterestList		PRESENCE optional		}|

{ ID id-LocationReportingRequestType		CRITICALITY ignore	TYPE LocationReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

Next Change

UserLocationInformation ::= CHOICE {

userLocationInformationEUTRA	UserLocationInformationEUTRA,

userLocationInformationNR		UserLocationInformationNR,

userLocationInformationN3IWF	UserLocationInformationN3IWF,

...

}

UserLocationInformationEUTRA ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

tAI					TAI,

timeStamp			Timestamp		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UserLocationInformationEUTRA-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationEUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationN3IWF ::= SEQUENCE {

iPAddress			TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationN3IWF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			Timestamp		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}