3GPP TSG-RAN WG3 Meeting #101	R3-184943

# Gothenburg, Sweden, 20th- 24th August  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1324 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of incorrect acronym introduced in QMC support for MTSI service type in UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MTSI service type is incorrectly named as MSTI in Message defination and ASN.1 IE defination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MSTI is replaced with MTSI in ‘Initial UE message’, ‘UE application layer message configuration’ and ‘UE application layer measurement configuration for relocation’ message and IE definations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect acronym for service type name would remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.33, 9.2.1.127, 9.2.1.129, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |

------------ Start of change ------------

### 9.1.33	INITIAL UE MESSAGE

This message is sent by the RNC to transfer the radio interface initial layer 3 message to the CN.

Direction: RNC  CN.

Signalling bearer mode: Connection oriented.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| CN Domain Indicator | M |  | 9.2.1.5 |  | YES | ignore |
| LAI | M |  | 9.2.3.6 |  | YES | ignore |
| RAC | C - ifPS |  | 9.2.3.7 |  | YES | ignore |
| SAI | M |  | 9.2.3.9 |  | YES | ignore |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | ignore |
| Iu Signalling Connection Identifier | M |  | 9.2.1.38 |  | YES | ignore |
| Global RNC-ID | M |  | 9.2.1.39 | If the Extended RNC-ID IE is included in the message, the RNC-ID IE in the Global RNC-ID IE shall be ignored. | YES | ignore |
| GERAN Classmark | O |  | 9.2.1.57 |  | YES | ignore |
| Selected PLMN Identity | O |  | 9.2.3.33 |  | YES | ignore |
| NAS Sequence Number | O |  | 9.2.3.34 |  | YES | ignore |
| Permanent NAS UE Identity | O |  | 9.2.3.1 |  | YES | ignore |
| Redirect Attempt Flag | O |  | 9.2.3.50 |  | YES | ignore |
| Extended RNC-ID | O |  | 9.2.1.39a | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. | YES | reject |
| CSG Id | O |  | 9.2.1.85 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.93 |  | YES | reject |
| L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to LIPA operation. | YES | ignore |
| Higher bitrates than 16 Mbps flag | O |  | 9.2.3.54 | May only be included towards the PS domain. | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.6 | Indicating HNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to SIPTO@LN operation. | YES | ignore |
| LHN ID | O |  | 9.2.1.123 |  | YES | ignore |
| SGSN Group Identity | O |  | 9.2.3.57 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| DCN ID | O |  | INTEGER (0..65535) |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MTSI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifPS | This IE shall be present if the CN Domain Indicator IE is set to "PS domain". |

------------ Skip to next change ------------

#### 9.2.1.127	UE Application layer measurement configuration

The purpose of the UE Application layer measurement configuration IE is to provide configuration information for the QoE function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) |  | Yes | ignore |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | Yes | ignore |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | ignore |

------------ Skip to next change ------------

#### 9.2.1.129	UE Application layer measurement configuration for relocation

The purpose of the UE Application layer measurement configuration for relocation IE is to provide configuration information for the QoE function during relocation procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | YES | ignore |
| Trace Reference | M |  | 9.2.1.8 |  | YES | ignore |
| Trace Propagation Parameters | O |  | 9.2.1.68 | Optional for UTRAN. Not applicable to GERAN Iu Mode. | YES | ignore |
| Trace Collection Entity IP Address | O |  | 9.2.2.1 |  | YES | ignore |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | ignore |

------------ Skip to next change ------------

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

------------ Skip to next change ------------

UE-Application-Layer-Measurement-Configuration	::= SEQUENCE {

applicationLayerContainerForMeasurementConfiguration		OCTET STRING ( SIZE (1..1000)),

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

...,

serviceType													ServiceType

}

UE-Application-Layer-Measurement-Configuration-For-Relocation	::= SEQUENCE {

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

traceReference						TraceReference,

tracePropagationParameters			TracePropagationParameters OPTIONAL,

traceCollectionEntityIPAddress		TransportLayerAddress			OPTIONAL,

...,

serviceType							ServiceType

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

...

}

------------ Skip to next change ------------

TargetRNC-ToSourceRNC-TransparentContainer-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

{ ID id-UeApplicationLayerMeasurementSupportIndication	CRITICALITY ignore	EXTENSION  UeApplicationLayerMeasurementSupportIndication		PRESENCE optional},

...

}

UeApplicationLayerMeasurementSupportIndication::= BIT STRING (SIZE (8))

-- First bit: QoE Measurement for streaming service

-- Second bit: QoE Measurement for MTSI service

-- Note that undefined bits are considered as a spare bit and spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.

TBCD-STRING 				::= OCTET STRING

TemporaryUE-ID ::= CHOICE {

tMSI				TMSI,

p-TMSI				P-TMSI,

...

}

------------ Skip to next change ------------

UE-Application-Layer-Measurement-Capability ::= BIT STRING (SIZE (8))

-- First bit: QoE Measurement for streaming service

-- Second bit: QoE Measurement for MTSI service

-- Note that undefined bits are considered as a spare bit and spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.

UE-History-Information ::= OCTET STRING

------------ End of change ------------