3GPP TSG-RAN WG3 Meeting #101	R3-185029

# Gothenburg, Sweden, 20th- 24th August  2018

Agenda Item:	31.3.4.3

Source:	Ericsson

Title: 	The Maximum Data Burst Volume presence discussion

Document for:	Discussion and Approval

# 1	Discussion

At the last meeting, there is a proposal to clarify that the Maximum Data Burst Volume IE only applies delay critical GBR QoS flows.

The implemetnation of this proposal is to change the presence of the IE from Optional to Conditional ( Condition: the presence of Delay Critical).

From the stage 2, it is very clear that the QoS characteristics “Maximum Data Burst Volume” is for Delay-critical GBR resource type only.

However to change the presence of the Maximum Data Burst Volume from Optional to Conditional is incorrect!

This change would cause that the “Maximum Data Burst Volume” shall always present when the “Delay Critical” IE is presneted, that this is NOT true.

Observation 1: To change the Maximum Data Burst Volume IE to conditional would make the IE mandatory present for Delay Critical GBR QoS flows.

Read from TS 23.501 v.15.2.0:

#### 5.7.3.7	Maximum Data Burst Volume

Each GBR QoS Flow with Delay-critical resource type shall be associated with a Maximum Data Burst Volume (MDBV).

MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB (i.e. 5G-AN part of the PDB). The MDBV may be signalled with 5QIs to the (R)AN, and if it is not received, a standardized value applies (for standardized 5QIs the value in the QoS characteristics Table 5.7.4 applies).

Observation 2: The Maximum Data Burst Volume is for Delay Critical GBR QoS Flow only, but it does not mean it shall always be signlled to RAN.

The current TS 38.413 and the other spec where QoS parameters are deifned has the below strucutre or similar:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | 9.3.1.84 | Priority Level is specified in TS 23.501 [9]. |
| Packet Delay Budget | M |  | 9.3.1.80 | Packet Delay Budget is specified in TS 23.501 [9]. |
| Packet Error Rate | M |  | 9.3.1.81 | Packet Error Rate is specified in TS 23.501 [9]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | Indicates whether the GBR QoS flow is delay critical as specified in TS 23.501 [9]. |
| Averaging Window | C-ifGBRflow |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. |
| Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. |

The above table contains the 6 QoS characteristics defined by SA2 and has been well discussed.

It is specified that “Maximum Data Burst Volume is specified in TS 23.501,” where the presence etc of the IE is well specified already.

Proposal 1	RAN3 to agree not to change the presence of the Maximum Data Burst Volume IE.

# 2	Conclusion

Proposal 1	RAN3 to agree not to change the presence of the Maximum Data Burst Volume IE.