3GPP TSG-RAN WG3#101bis	R3-185361

Chengdu, China, 8 - 12 October 2018

Agenda item:	10.5.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.300): TP for option 3 in Inter-system handover

Document for:	Discussion and Decision

# 1	Introduction

The handling of end marker packets has been addressed in previous RAN3 meetings.

This paper focusses on the remaining inter-system scenario. It tries to build the best solution taking into account both the NG-RAN impacts and the UPF impacts.

# 4	Text Proposal for TS 38.300

### 9.3.2	From 5GC to EPC

[Unchanged text skipped]

#### 9.3.2.4	Data Forwarding

The inter-System data forwarding follows the following key principles:

-	Only indirect data forwarding is supported.

-	PDU session information at the serving NG-RAN node contains mapping information per QoS Flow to a corresponding E-RAB.

-	At handover preparation, the source NG-RAN node shall decide which mapped E-RABs are proposed to be subject to data forwarding and provide this information in the source-to-target container to the target eNB.

-	The target eNB assigns forwarding TEID/TNL address(es) for the E-RAB(s) for which it accepts data forwarding.

-	A data forwarding tunnel is established between the source NG-RAN node and UPF per E-RAB which  is subject to data forwarding. For the QoS flow(s) accepted for data forwarding, the NG-RAN node initiates data forwarding to the UPF over the corresponding E-RAB tunnel(s). Then the UPF maps data received from the E-RAB tunnel  to the mapped EPS bearer(s).

### 9.3.3	From EPC to 5GC

#### 9.3.3.1	Data Forwarding

The inter-System data forwarding from EPS to 5GS follows the following key principles:

-	Only indirect data forwarding is supported.

-	The target NG-RAN node receives in the Handover Request message the mapping between E-RAB ID(s) and QoS Flow ID(s). It decides whether to accept the data forwarding for E-RAB IDs proposed for forwarding within the source to target container. It assigns a TEID/TNL address for each E-RAB involved in the accepted forwarding.

-	The target NG-RAN node sends the Handover Request Acknowledge message in which it indicates the list of PDU sessions and QoS flows for which it has accepted the forwarding.

-	The source eNB receives in the Handover Command message the list of E-RAB IDs for which the target NG-RAN node has accepted the forwarding of corresponding PDU sessions and QoS flows.

-	For each E-RAB accepted for data forwarding, the source eNB forwards data to the SGW in the corresponding E-RAB tunnel and the SGW forwards the received data to the UPF over the EPS bearer tunnel. Then the UPF maps the data received from an EPS bearer tunnel to the corresponding E-RAB tunnel. The target NG-RAN node prioritizes the forwarded packets over the fresh packets for those QoS flows which are involved in the accepted forwarding.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-183611, Handling end marker packets in 5GS intra-system