3GPP TSG-RAN WG3 #101bis																	       R3-185485

Chengdu, China, 8-12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1236 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for LS on RLC mode reconfiguration in 36.423 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the LS R2-1813220, RAN2 agreed that: “RLC mode reconfiguration can only be supported by DRB release/addition (including RLC bearer release) or full configuration.This also means that, in a change of the PDCP termination point (e.g. a bearer type change from an MN terminated bearer to an SN terminated bearer), the RLC mode originally determined by the old hosting node needs to be known at the new one, to avoid the risk that a different RLC mode is selected and that the DRB must be released.” This CR is based on this LS from RAN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of e.g., a bearer type change from an MN terminated bearer to an SN terminated bearer, a new IE “Old RLC Mode” is introduced in the “E-RABs To Be Added Item” IE included in the SGNB ADDITION REQUEST message and SGNB MODIFICATION REQUEST message, in order to indicate the RLC mode originally determined by the Maste node. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol and functional point of view. This CR is non-backwards compatible The impact can be considered isolated to SGNB ADDITION REQUEST and SGNB MODIFICATION REQUEST. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | According to the current X2 signalling, the RLC mode is determined by the PDCP anchor node, and it is not possible for the SN to avoid the reconfiguration of RLC mode for one DRB in case of bearer type change from MN terminated bearer to SN terminated bearer, due to the lack of information about the RLC mode used in MN before. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1, 9.1.4.5, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

9.1.4.1	SGNB ADDITION REQUEST

This message is sent by the MeNB to the en-gNB to request the preparation of resources for EN-DC operation for a specific UE

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| NR UE Security Capabilities | M |  | 9.2.107 |  | YES | reject |
| SgNB Security Key | M |  | 9.2.101 | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. | YES | reject |
| SgNB UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.12 | The UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SgNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and en-gNB respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.4 | The selected PLMN of the SCG in the en-gNB. | YES | ignore |
| Handover Restriction List | O |  | 9.2.3 |  | YES | ignore |
| E-RABs To Be Added List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DRB ID | M |  | 9.2.122 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>Full E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB level QoS parameters as received on S1-MME. | – |  |
| >>>>Maximum MCG admittable E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBR |  | GBR QoS Information 9.2.10 | Includes the GBR QoS Information admittable by the MCG. | – |  |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>MeNB DL GTP TEID at MCG | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>> RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode originally determined by the MeNB in case of bearer type change. | – |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>Requested SCG E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>MeNB UL GTP TEID at PDCP | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>Secondary MeNB UL GTP TEID at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>UL configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>PDCP SN Length | O |  | 9.2.133 | Indicates the PDCP SN length of the bearer. | YES | reject |
| MeNB to SgNB Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Requested split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested. | YES | reject |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between MeNB and en-gNB. | YES | ignore |
| SGNB Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-eNB HO, intra-eNB HO, ...) | This IE indicates the trigger for SGNB Addition procedure. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | YES | ignore |
| MeNB Cell ID | M |  | ECGI 9.2.14 | Indicates the cell ID for PCell in MeNB. | YES | reject |

<<<<<< NEXT CHANGE >>>>>>

9.1.4.5	SGNB MODIFICATION REQUEST

This message is sent by the MeNB to the en-gNB to request the preparation to modify en-gNB resources for a specific UE, to query for the current SCG configuration, or to provide the S-RLF-related information to the en-gNB.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.4 | The selected PLMN of the SCG in the en-gNB. | YES | ignore |
| Handover Restriction List | O |  | 9.2.3 |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.103 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >NR UE Security Capabilities | O |  | 9.2.107 |  | – |  |
| >SgNB Security Key | O |  | 9.2.101 |  | – |  |
| >SgNB UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.12 |  | – |  |
| >E-RABs To Be Added List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Added Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>DRB ID | M |  | 9.2.122 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". | – |  |
| >>>>>Full E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters as received on S1-MME. | – |  |
| >>>>>Maximum MCG admittable E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBR |  | GBR QoS Information 9.2.10 | Includes the GBR QoS Information admittable by the MCG. | – |  |
| >>>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>>MeNB DL GTP TEID at MCG | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>> RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode originally determined by the MeNB in case of bearer type change. | – |  |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>>Requested SCG E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes necessary E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>>MeNB UL GTP TEID at PDCP | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>>Secondary MeNB UL GTP TEID at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>>UL configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>>PDCP SN Length | O |  | 9.2.133 | Indicates the PDCP SN length of the bearer. | YES | ignore |
| >E-RABs To Be Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>>Full E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters to be modified as received on S1-MME | – |  |
| >>>>>Maximum MCG admittable E-RAB Level QoS Parameters | O |  | GBR QoS Information 9.2.10 | Includes the GBR QoS information admittable by the MCG | – |  |
| >>>>>MeNB GTP TEID at MCG | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>>S1 UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established.. |  |  |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>>Requested SCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>>MeNB UL GTP TEID at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>>UL configuration | O |  | 9.2.118 | Information about UL usage in the en-gNB. | – | – |
| >>>>>PDCP SN Length | O |  | 9.2.133 | Indicates the PDCP SN length of the bearer. | YES | ignore |
| >E-RABs To Be Released List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Released Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of UL PDUs | – |  |
| >>>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | - | - |
| MeNB to SgNB Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between MeNB and en-gNB. | YES | ignore |
| Requested split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested. | YES | ignore |
| Requested split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested to be released. | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

9.3	Message and Information Element Abstract Syntax (with ASN.1)

<Skip unchanged part>

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-AerialUEsubscriptionInformation,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-RLC-Status,

id-rlc-Mode,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

<Skip unchanged part>

-- **************************************************************

--

-- SGNB ADDITION REQUEST

--

-- **************************************************************

SgNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container {{SgNBAdditionRequest-IEs}},

...

}

SgNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-NRUESecurityCapabilities					CRITICALITY reject	TYPE NRUESecurityCapabilities					PRESENCE mandatory}|

{ ID id-SgNBSecurityKey							CRITICALITY reject	TYPE SgNBSecurityKey									PRESENCE mandatory}|

{ ID id-SgNBUEAggregateMaximumBitRate			CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-SelectedPLMN								CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-SgNBAddReqList			CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReqList			PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY reject	TYPE SplitSRBs										PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-SGNB-Addition-Trigger-Ind				CRITICALITY reject	TYPE SGNB-Addition-Trigger-Ind					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP				CRITICALITY ignore	TYPE SubscriberProfileIDforRFP					PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY reject	TYPE ECGI											PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBAddReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBAddReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBAddReq-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBAddReq-Item		CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID								DRB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters				E-RAB-Level-QoS-Parameters,

max-MCG-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in  Full E-RAB Level QoS Parameters IE --

dL-Forwarding								DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG						GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint						GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-rlc-Mode							CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint	OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration	OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs} }		 OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-pDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

<Skip unchanged part>

-- **************************************************************

--

-- SGNB MODIFICATION REQUEST

--

-- **************************************************************

SgNBModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ SgNBModificationRequest-IEs}},

...

}

SgNBModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-SelectedPLMN						CRITICALITY ignore	TYPE PLMN-Identity								PRESENCE optional}|

{ ID id-HandoverRestrictionList				CRITICALITY ignore	TYPE HandoverRestrictionList					PRESENCE optional}|

{ ID	id-SCGConfigurationQuery			CRITICALITY ignore  TYPE SCGConfigurationQuery						PRESENCE optional}|

{ ID id-UE-ContextInformation-SgNBModReq	CRITICALITY reject	TYPE UE-ContextInformation-SgNBModReq		PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer					CRITICALITY ignore	TYPE MeNBtoSgNBContainer						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation	CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation	PRESENCE optional}|

{ ID id-RequestedSplitSRBs					CRITICALITY ignore	TYPE SplitSRBs									PRESENCE optional}|

{ ID id-RequestedSplitSRBsrelease			CRITICALITY ignore	TYPE SplitSRBs									PRESENCE optional},

...

}

UE-ContextInformation-SgNBModReq ::= SEQUENCE {

nRUE-SecurityCapabilities		NRUESecurityCapabilities															OPTIONAL,

sgNB-SecurityKey				SgNBSecurityKey 																	OPTIONAL,

sgNBUEAggregateMaximumBitRate	UEAggregateMaximumBitRate															OPTIONAL,

e-RABs-ToBeAdded				E-RABs-ToBeAdded-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeModified				E-RABs-ToBeModified-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeReleased				E-RABs-ToBeReleased-SgNBModReq-List													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UE-ContextInformationSgNBModReqExtIEs} }			OPTIONAL,

...

}

UE-ContextInformationSgNBModReqExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{	ID id-SubscriberProfileIDforRFP					CRITICALITY ignore	EXTENSION SubscriberProfileIDforRFP									PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBModReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBModReq-Item		CRITICALITY ignore	TYPE E-RABs-ToBeAdded-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID									DRB-ID,

en-DC-ResourceConfiguration				EN-DC-ResourceConfiguration,

resource-configuration					CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in  Full E-RAB Level QoS Parameters IE --

dL-Forwarding							DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-rlc-Mode							CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint		OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration																OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-pDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-SgNBModReq-ItemIEs} }

E-RABs-ToBeModified-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeModified-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeModified-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																		OPTIONAL,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																				OPTIONAL,

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional },

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters																			OPTIONAL,

meNB-UL-GTP-TEIDatPDCP						GTPtunnelEndpoint																					OPTIONAL,

uL-Configuration								ULConfiguration																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-pDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBModReq-ItemIEs} }

E-RABs-ToBeReleased-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

<Skip unchanged part>

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-dL-Forwarding															ProtocolIE-ID ::= 307

id-E-RABs-DataForwardingAddress-List										ProtocolIE-ID ::= 308

id-E-RABs-DataForwardingAddress-Item										ProtocolIE-ID ::= 309

id-Subscription-Based-UE-DifferentiationInfo								ProtocolIE-ID ::= 310

id-rlc-Mode																	ProtocolIE-ID ::= xxx

<<<<<< End of CHANGE >>>>>>