3GPP TSG-RAN WG3 Meeting #101bis	R3-185510

Chengdu, China, Oct. 8th – 12th, 2018

Agenda item:		23.2.

Source:	LG Electronics Inc.

Title:	Issues to be considered on NR V2X

Document for:		Discussion

1. Introduction

A new Study Item on NR V2X was approved in RAN #80 and updated in RAN #81 [1], and the corresponding time budget was allocated for this Study Item with completion date of March 2019 [2].

As the first meeting in RAN3 for this SI, this paper is to investigate the potential issues of RAN3 for this item and the corresponding proposals are also given.

2. Discussion

In Rel-14 LTE based V2X, on service authorization we have specified the V2X Services Authorized Information, which includes Vehicle UE and Pedestrian UE, for notifying the authorization status of a specific UE. eNB can decide how to allocate the PC5 resources based on the authorization information received for a specific UE and also the RRC indication information from UE on UE type.

On the other hand, we have also specified a QoS function, i.e., introduced UE Sidelink Aggregate Maximum Bit Rate for assisting eNB to manage the use of sidelink radio resources. The aim is not to go over a limit for a specific UE.

For NR V2X, similar authorization function and QoS function are needed in RAN side since RAN1 is designing very similar resource modes on NR sidelink for V2X. This is the necessity of pure NR sidelink based authorization and UE sidelink AMBR.

Observation): Similar Authorization Function and QoS Function (NR UE Sidelink AMBR) are needed for NR (gNB case).

On the other hand, NG-RAN can be either gNB or ng-eNB, that is, from radio point of view, it is either NR or LTE. A UE may support NR or LTE or both. Sidelink radio resource management mechanisms are different for NR and LTE. It seems necessary to separate the information on V2X Services Authorized Indication / UE Sidelink AMBR from gNB and ng-eNB in order to adopt different resource modes for NR and LTE in radio part.

Thus, it requests AMF to provide V2X Services Authorized Information / UE Sidelink AMBR to gNB and ng-eNB separately.

In addition, SA2 has discussed Service Authorization to NG-RAN for eV2X communications over PC5 reference point and the related solution captured in clause 6.6 “Solution #6: eV2X impacts to 5GC procedures” of TR 23.786 [3], which is also given as follows:

6.6.2.6 V2X capability indication and V2X related information per PC5 RAT A UE may support multiple radio access technologies (RATs) over PC5 reference point, including LTE and NR. For such UE, the V2X capability indication and V2X related information can be per PC5 RAT as below: - The V2X capability indication sent by the UE described in clause 6.6.2.1 is per PC5 RAT. The V2X capability per PC5 RAT indicates whether the UE is capable of supporting V2X communication over PC5 reference point, i.e. over LTE PC5, over NR PC5 or over both. - The "V2X services authorized" indication sent to NG-RAN described in clause 6.6.2.1 to clause 6.6.2.5 is per PC5 RAT. - The UE-PC5-AMBR sent to NG-RAN described in clause 6.6.2.1 to clause 6.6.2.5 is per PC5 RAT. NOTE: For V2X capability indication and V2X related information per PC5 RAT, coordination with RAN WGs is needed.

Based on the analysis above, the following proposals are suggested to RAN3:

Proposal 1): The “V2X Services Authorized” Information sent to NG-RAN is per PC5 RAT.

Proposal 2): The “UE Sidelink Aggregate Maximum Bit Rate” sent to NG-RAN is per PC5 RAT.

3. Conclusion

In this contribution, the potential issues on NR V2X SI were investigated. The following proposals are suggested to RAN3:

Proposal 1): The “V2X Services Authorized” Information sent to NG-RAN is per PC5 RAT.

Proposal 2): The “UE Sidelink Aggregate Maximum Bit Rate” sent to NG-RAN is per PC5 RAT.

4. References

- RP-182111, “Study on NR V2X”, LG Electronics

- RP-182040, “RAN3 current time budget planning”, RAN3 Chairman

- 3GPP TR 23.786 V0.8.0: Study on architecture enhancements for EPS and 5G System to support advanced V2X services