3GPP TSG RAN WG3 Meeting #101bis	R3-185546

Chengdu/China, 8-12th October 2018

Title:	NTN - Min altitude for LEO based scenario

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2 Architecture

Study item:	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Abstract

This technical contribution aims at proposing modifications to the in chapter 4.2. “Non-Terrestrial Networks reference scenarios” of the technical report related to 3GPP TR 38.821 “Study on solutions for NR to support non terrestrial networks (NTN)”.

# Rational

For scenario C& D have been adopted corresponding to non terrestrial network based on respectively transparent or regenerative satellites.

Currently 2 altitudes are proposed. 600 and 1200 km.

In order to reduce the number of options to be studied, it is proposed here to only keep the 600 km which is the worst case in terms of Doppler and mobility.

# Proposed Text

We suggest the following modifications.

* * * Start of changes * * * * (MODIFIED TEXT)

## 4.2	Non-Terrestrial Networks reference scenarios

We shall consider in this document non-terrestrial networks providing access to user equipment in two reference scenarios including

- Circular orbiting and notional station keeping platforms.

- Highest RTD constraint

- Highest Doppler constraint

- A transparent and a regenerative payload

- One ISL case and one without ISL. Regenerative payload is mandatory in the case of inter-satellite links.

Four scenarios are considered as depicted in Table 4.2-1 and are detailed in Table 4.2-2.

Table 4.2-1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network | Scenario C | Scenario D |

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km minimum |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth (service link) | 30 MHz for band < 6 GHz 400 MHz for band > 6 GHz |  |
| Payload | Scenario A : Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes |
| Earth-fixed beams | Yes | Scenario C: No (the beams move with the satellite) Scenario D, option 1: Yes (steering beams), see note 1 Scenario D, option 2: No (the beams move with the satellite) |
| Max beam foot print diameter at nadir | 500 km | 200 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° | 10° |
| Max distance between satellite and user equipment at min elevation angle | 40,586 km | 1,932 km (600 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 562 ms (service and feeder links) Scenario B: 281ms | Scenario C: 25.76 ms (transparent payload: service and feeder links) Scenario D: 12.88 ms (regenerative payload: service link only) |
| Max delay variation within a beam (earth fixed user equipment) | 16ms | 4.44ms (600km) |
| Max differential delay within a beam | 1.6 ms | 0.65 ms (*) |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (*) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (*) |
| User equipment motion on the earth | 1000 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1000 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 4 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir

* * * End of changes * * * *