3GPP TSG RAN WG3 Meeting #101bis	                                        R3-185559

Chengdu, China, 8-12 October 2018

Source:	CATT

Title:	Discussion on NR eV2X

Agenda Item:	23.4

Document for:	Discussion and Decision

Introduction

The new Study Item on Study on NR V2X SI [1] was agreed in RAN#80. RAN2 and RAN3 start the discussion on this SI from this meeting. In this contribution we analysis some issues for NR V2X which may introduce potential impacts on RAN3 work.

Discussion

1. Service Authorization for eV2X communications over PC5 reference point:

Some key issues captured in SA2 TR23.786 [2] are about service authorization for eV2X communications over PC5 interface. One use case is when the UE connects to NR, the service authorization information for NR eV2X communication over PC5 interface needs to be provided to NG-RAN from 5GC, e.g. assisting NG-RAN to operate in Mode 3 or 4, which is similarly with LTE V2X.

Another use case is when the UE connects to EPC, the UE may be allowed to use NR based PC5 communication, the MME may need to authorize the UE to use V2X service operation for NR PC5.

In order to support above two cases, the potential RAN3 impact may include:

-NG procedure enhancement: Registration procedure, service request procedure, NG handover procedure

-Xn procedure enhancement: Xn handover procedure

-F1 procedure enhancement: UE context Management procedure

-S1 procedure enhancement：Registration procedure, service request procedure, NG handover procedure

-X2 procedure enhancement: X2 handover procedure

Proposal 1: It’s proposed to consider the service authorization issue for the further RAN3 study.

2. Interworking operation for eV2X

The NR eV2X system needs to support both the V2X requirements and the eV2X requirements which mean the 5G system needs to handle all types of V2X UEs, e.g. UE with EPS V2X capabilities, and UE with 5GS V2X capabilities. To achieve maximum gain through the use of V2X application, the communication between the EPS V2X capable UE and 5GS V2X capable UE should be supported as much as possible, e.g. 5GS V2X capable UE's may need to communicate with EPS V2X capable UE for the common services, e.g. basic safety related V2X services.

The Example scenario of mixed 3GPP RATs deployment is shown in the Figure1. In the No coverage scenario, since there is no LTE and NR coverage, in order to support the communication between the 5GS V2X capable UE and EPS V2X capable UE, the PC5 interface will be selected as the data path. The related radio resources, policy and parameters for the PC5 operation need to be provided to the UE.

In the LTE only coverage scenario, if the EPS V2X capable UE uses the Uu interface for the V2X transmission, in this area, the 5GS V2X capable UE uses PC5 interface to communicate with other V2X UE, the transmission from the 5GS V2X capable UE may generate interference to the E-UTRAN system. In this case, if the 5GS V2X capable UE obtains the LTE cell related configuration parameters, the interference to the E-UTRAN might be minimized.

Figure1 Example scenario of mixed 3GPP RATs deployment

Proposal 2: It’s proposed to consider the interworking operation issue for the further RAN3 study.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It’s proposed to consider the service authorization issue for the further RAN3 study.

Proposal 2: It’s proposed to consider the interworking operation issue for the further RAN3 study.

References

# [1] RP-181480, New SID: Study on NR V2X,

[2] TR 23.786 v0.8.0, Study on architecture enhancements for EPS and 5G System to support advanced V2X services