3GPP TSG RAN WG3 NR#101bis                                                                    R3-185608

# Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	31.3.4.14

Source:	ZTE

Title:	(TP for NR BL CR for TS 38.300): Correction of RRC Inactive Assistant Information

Document for:	Discussion and Decision

1	Introduction

RAN2#103bis has agreed to use 5G-S-TMIS instead of IMSI as UE ID for paging, the updated TS38.304 related to UE-ID captured below.

38.304 v15.1.0 UE_ID: 5G-S-TMSI mod 1024 If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above. 5G-S-TMSI is a 48 bit long bit string as defined in [10]. 5G-S-TMSI shall in the formula above be interpreted as a binary number where the left most bit represents the most significant bit

Since UE Identity Index value in UE Identity Index value is designed to convey IMSI information from 5GC to NG-RAN, then this IE should be replaced by 5G-S-TMSI or UE Paging Identity.

2	Text Proposal for TS 38.300

9.2.2	Mobility in RRC_INACTIVE

9.2.2.1	Overview

[Unchanged text skipped]

The AMF provides to the NG-RAN node the RRC Inactive Assistant Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE. The RRC Inactive Assistant Information includes the registration area configured for the UE, the UE specific DRX, Periodic Registration Update timer, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, and UE Paging Identity. The UE registration area is taken into account by the NG-RAN node when configuring the RAN-based notification area. The UE specific DRX and UE Paging Identity are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RAN Notification Area Update timer.

References

- TS 38.304 v15.1.0,User Equipment (UE) procedures in Idle mode and RRC Inactive state