3GPP TSG-RAN WG3#101bis                                                                     R3-185645

Chengdu, China, 8-12 Oct 2018

Agenda Item:	10.8.4.1

Source:	CATT

Title:	(TP for BL CR for TS 38.423) Introduction of Xn-U TEID in S-Node

Modification Required message

Document for:	Discussion, Decision

1. Introduction

In EN-DC,it is agreed that SgNB may inform MeNB of the changed DL tunnel ID for X2-U.In this contribution,we propose to support this function for option 7.

2. Discussion

In EN-DC,to support SN initiated inter-SgNB-DU mobility, SgNB DL GTP TEID at SCG is introduced in SgNB Modification Required message which indicates the updated DL TEID for X2-U. So, similarly, for  option 7,the same information should be provided from SN to MN during SN initiated inter-SgNB-DU mobility procedure. Based on that, we have the following proposal:

Proposal 1: It is proposed to introduce Xn DL UP TNL Information related IE for MN terminated bearers in S-NODE MODIFICATION REQUIRED message.

3. Conclusion

The following proposal is made:

Proposal 1: It is proposed to introduce Xn DL UP TNL Information related IE for MN terminated bearers in S-NODE MODIFICATION REQUIRED message.

4. TP for 38.423

<<<<<<<<<<<<<<<<<<<<Start of the first change >>>>>>>>>>>>>>>>>>>>

9.2.1.22	PDU Session Modification Required Info – MN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains PDU session resource information of an S-NG-RAN node initiated modification request of DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs To Be Released List | O |  | DRB List with Cause 9.2.1.28 |  | – |  |
| DRBs To Be Modified List | O |  |  |  |  |  |
| > DRBs To Be Modified Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |  |  |
| >> SN DL PDCP UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer. For delivery of DL PDUs. |  |  |
| >> secondary SN DL PDCP UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer. For delivery of DL PDUs in case of PDCP Duplication |  |  |
| >> LCID | O |  | 9.2.3.70 | LCID for primary path if PDCP duplication is applied |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is 32. |

<<<<<<<<<<<<<<<<<<<<Second change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- PDU Session Resource Modification Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-MNterminated ::= SEQUENCE {

dRBsToBeReleased				DRB-List-withCause									OPTIONAL,

dRBsToBeModified                DRBsToBeModified-List-ModRqd-MNterminated           OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-MNterminated-Item

DRBsToBeModified-List-ModRqd-MNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportLayerInformation	,

secondary-SN-DL-SCG-UP-TNLInfo		UPTransportLayerInformation		OPTIONAL,

lCID                                    LCID                            OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModRqdInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<End of the change >>>>>>>>>>>>>>>>>>>>