3GPP TSG-RAN WG3 Meeting #101bis	     R3-185681

Chengdu, P.R. China, 8th - 12th October 2018

Agenda Item:	22.2

Source:	Ericsson

Title:	gNB identification and grouping in RIM scenarios

Document for:	Discussion and agreement

# Introduction

RAN1 sent an LS to RAN3 (R3-185441), asking for feedback on the feasibility of the three candidate RIM solution frameworks (FW-1, FW-2.1 and FW-2.2).

This paper discusses the issue of gNB grouping and identification in remote interference (RI) scenarios, which is a vital part of all three candidate frameworks.

# Discussion

Identification and grouping of gNBs in RIM candidate frameworks (FW) are discussed below.

2.1 gNB sets and RIM reference signals

At the RAN1#94 meeting it was agreed that the gNBs in an RI scenario may be grouped into gNB sets where each gNB set is affiliated to a different RIM reference signal (RS). Depending on the RI scenario and network deployment, a set can comprise one or more gNBs. For example, in a scenario with many involved gNBs, the gNBs can be grouped in sets based on e.g. that they experience or cause similar RI characteristics. The current common understanding is that all gNBs belonging to the same set transmit identical reference signals (RS) and map the RS to the physical resources in the same manner. Furthermore, it is the current common understanding that the RS conveys the ID of its corresponding gNB set.

Observation 1: There can exist RI scenarios where a group of gNBs experience, or cause, similar RI characteristics, and are hence suitable for grouping.

Proposal 1: Specify grouping of gNBs into sets.

2.2 gNB set formation

As mentioned earlier, the concept of gNB sets may be applicable to scenarios with both small and large number of gNBs involved. As the RI scenario properties and affected gNB population may vary from one RI occasion to another, it does not seem meaningful to apply fixed (i.e. preconfigured or OAM-based) grouping of gNBs into sets. This calls for an adaptive mechanism for assignment of gNB set IDs to individual gNBs.

In an RI scenario comprising many gNBs, it is obvious that self-grouping mechanisms are not feasible. Instead, a meaningful approach is to rely on assistance from the 5GC in gNB set formation. The assistance would be provided by a RIM-facilitating 5GC functional entity. This role could be performed by, for example, the AMF. The sequence of steps may look as follows:

- A number of gNBs detect RI

- These gNBs notify the RIM-facilitating 5GC functional entity (e.g. the AMF) that they have detected RI

- The RIM-facilitating 5GC functional entity divides the involved gNBs into gNB sets and notifies the involved gNBs about their assigned set ID

- Based on their assigned set IDs, the gNBs infer their respective RS configurations and may transmit the RSs, according to the RS configuration

Proposal 2: RAN3 to agree on 5GC-assisted gNB set formation.

The concept of 5GC-assisted gNB set formation is illustrated in Figure 1.

Figure 1: An example of 5GC-assisted gNB set formation. The AMF 1 oversees gNB sets A-M, while AMF 2 oversees gNB sets N-Q

2.3 The need for gNB set formation in all candidate RIM frameworks

As both backhaul- and non-backhaul-based candidate RIM frameworks rely on the use of RS, it follows that they all require a mechanism for gNB set formation.

Observation 2: Both backhaul- and non-backhaul-based candidate RIM frameworks rely on the concept of gNB sets and they all require a mechanism for gNB set formation.

Considering the above, it is desirable to support gNB set formation based on a unified design.

Proposal 3: RAN3 to agree to a single mechanism for gNB set formation that could apply to all RIM candidate solutions.

# Conclusion

This paper discusses the issue of gNB grouping in remote interference scenarios and raises the following observations and proposals:

Observation 1: There can exist RI scenarios where a group of gNBs experience, or cause, similar RI characteristics, and are hence suitable for grouping.

Observation 2: Both backhaul- and non-backhaul-based candidate RIM frameworks rely on the concept of gNB sets and they all require a mechanism for gNB set formation.

Based on the observations, the following proposals are raised:

Proposal 1: Specify grouping of gNBs into sets.

Proposal 2: RAN3 to agree on 5GC-assisted gNB set formation.

Proposal 3: RAN3 to agree to a single mechanism for gNB set formation that could apply to all RIM candidate solutions.