3GPP TSG-RAN WG3 Meeting #101bis	     R3-185795

Chengdu, P.R. China, 8th - 12th October 2018

Agenda Item:	31.3.4.10

Source:	Ericsson

Title:	Radio quality information report triggering

Document for:	Discussion and agreement

# Introduction

This paper proposes the mechanism for triggering the Transfer of Assistance Information procedure only when needed. The accompanying CR to TS 38.425 is presented in R3-185796.

# Discussion

The TS 38.425 currently defines the Transfer of Assistance Information procedure, whose purpose is to provide assistance information to the node hosting the NR PDCP entity. However, the node hosting the NR PDCP entity does not have the possibility to request the Radio Quality Assistance Information only when needed. Consequently, it is possible that the corresponding node will start sending reports to the node hosting the NR PDCP entity, even in the case when the node hosting the NR PDCP entity does not need these reports. This will result in a waste of resources.

Observation: it is possible that the corresponding node will start sending reports to the node hosting the NR PDCP entity, even in the case when the node hosting the NR PDCP entity does not need these reports, which will result in a waste of resources.

It is therefore necessary to enable the node hosting the NR PDCP entity to set the criteria that will trigger the Transfer of Assistance Information procedure. Obviously, the triggering criteria should be in the form of threshold, which, when exceeded, triggers the procedure. Each threshold should be related to one of the measurables listed Assistance Information Types defined in the TS 38.425. If any of the thresholds for the Radio Quality Assistance Information is present in the DL USER DATA PDU and if the Radio Quality Assistance Information corresponding to the threshold present and measured at the corresponding node exceeds the threshold value, the corresponding node shall invoke the Transfer of Assistance Information procedure. The corresponding node shall include in the ASSISTANCE INFORMATION DATA each Radio Quality Assistance Information exceeding its corresponding threshold.

Proposal: RAN3 is respectfully asked to agree the CR to TS 38.425, presented in R3-185796.

# Conclusion

This paper discusses the triggering of Transfer of Assistance Information procedure in TS 38.425. The following observation was made:

Observation: it is possible that the corresponding node will start sending reports to the node hosting the NR PDCP entity, even in the case when the node hosting the NR PDCP entity does not need these reports, which results in resource wastage.

Based on the above observation, the following is proposed:

Proposal: RAN3 is respectfully asked to agree the CR to TS 38.425, presented in R3-185796.