3GPP TSG-RAN WG3 NR Meeting #101bis	R3-185856

# Chengdu, P.R. China, 8th- 12th October  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [TP BL CR TS 38.463] Asymmetric UL and DL QoS flows to DRB mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce Asymmetric UL and DL QoS flows to DRB mapping |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include in a new IE Asymmetric UL QoS Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Asymmetric UL and DL QoS flows to DRB mapping not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | ENUMERATED (DRB, PDU Session, UE, …) |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates to Suspend or Resume the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | M |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.5 |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >>>>S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | M |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | 9.3.1.20 | This IE shall be present when at least one non-GBR QoS Flows is being setup. | - | - |
| >>>>NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to PDU Session. | - | - |
| >>>>DRB To Setup List |  | 1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.25 | The UL QoS flow is not mapped to the DRB if the QoS Flow Indicator is included in the Not Mapped UL Flows Mapped to DRB Item IE included in the Asymmetric UL QoS Information IE. | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>Asymmetric UL QoS Information | O |  | 9.2.x.y | Contains asymmetric UL QoS information | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

------------------------

Skip to next change

------------------------

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates to Suspend or Resume the Bearer Context | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to UE. | - | - |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | M |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Trasnport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.5 |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | O |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.5 |  | - | - |
| >>>>PDCP Count Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP Count in the response message. | - | - |
| >>>>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>Cell Group To Add | O |  | 9.3.1.11 |  | - | - |
| >>>>Cell Group To Modify | O |  | 9.3.1.11 |  | - | - |
| >>>>Cell Group To Remove | O |  | 9.3.1.11 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >>>>S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | M |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | 9.3.1.20 | This IE shall be present when non-GBR QoS Flows are setting up. | - | - |
| >>>>NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to PDU Session. | - | - |
| >>>>DRB To Setup List |  | 1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.25 | The UL QoS flow is not mapped to the DRB if the QoS Flow Indicator is included in the Not Mapped UL Flows Mapped to DRB Item IE included in the Asymmetric UL QoS Information IE. | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>Asymmetric UL QoS Information | O |  | 9.2.x.y | Contains asymmetric UL QoS information | YES | ignore |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>PDU Session Type | O |  | 9.3.1.22 |  | - | - |
| >>>>S-NSSAI | O |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | O |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | 9.3.1.20 |  | - | - |
| >>>>NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to PDU Session. | - | - |
| >>>>DRB To Setup List |  | 0..1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.25 | The UL QoS flow is not mapped to the DRB if the QoS Flow Indicator is included in the Not Mapped UL Flows Mapped to DRB Item IE included in the Asymmetric UL QoS Information IE. | - | - |
| >>>>>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>Asymmetric UL QoS Information | O |  | 9.2.x.y | Contains asymmetric UL QoS information | YES | ignore |
| >>>>DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Overrides previous information. | - | - |
| >>>>>>PDCP Count Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP Count in the response message. | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>>>Cell Group To Add | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Modify | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Remove | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. The UL QoS flow is not mapped to the DRB if the QoS Flow Indicator is included in the Not Mapped UL Flows Mapped to DRB Item IE included in the Asymmetric UL QoS Information IE. | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivty Timer 9.3.1.54 | Included if the Activity Level Notification is set to DRB. | - | - |
| >>>>>>Asymmetric UL QoS Information | O |  | 9.2.x.y | Contains asymmetric UL QoS information | YES | ignore |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

------------------------

Skip to next change

------------------------

#### 9.2.x.y	Asymmetric UL QoS Information

This information element provides asymmetric UL QoS flow information for a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Not Mapped UL Flows to DRB Item |  | 0 .. <maxnoofQoSFlows> |  | Contains UL QoS flows which are not mapped to the DRB. |
| > QoS Flow Indicator | M |  | 9.3.1.24 |  |
| Asymmetric UL Flows Mapped to DRB Item |  | 0 .. <maxnoofQoSFlows> |  | Contains information of the UL QoS flows mapped to the DRB in asymmetric QoS flow to DRB mapping. |
| > QoS Flow Indicator | M |  | 9.3.1.24 |  |
| >Asymmetric UL QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.26 | Contains only QoS Flow level QoS parameters related to UL. The other IEs not related to UL are ignored. |

### 9.4.5	Information Element Definitions

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AsymmetricUL-QoS-Information,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

------------------------

Skip to next change

------------------------

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request	OPTIONAL,

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

qoS-Flow-List-PDU-Session-DL-Forwarding	QoS-Flow-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information-Response	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Response-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Response-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

------------------------

Skip to next change

------------------------

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDCP-Count-Request							PDCP-Count-Request						OPTIONAL,

pDCP-UL-Count								PDCP-Count								OPTIONAL,

pDCP-DL-Count								PDCP-Count								OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AsymmetricUL-QoS-Information		CRITICALITY ignore	EXTENSION AsymmetricUL-QoS-Information		PRESENCE optional },

...

}

AsymmetricUL-QoS-Information ::= SEQUENCE {

notMappedUL-FlowsTo-DRB-Item					NotMappedUL-FlowsTo-DRB-Item				OPTIONAL,

asymmetricUL-Flows-MappedToDRB-Item				AsymmetricUL-Flows-MappedToDRB-Item			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { AsymmetricUL-QoS-Information-ItemExtIEs } }	OPTIONAL,

...

}

AsymmetricUL-QoS-Information-ItemExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

NotMappedUL-FlowsTo-DRB-Item	::=	SEQUENCE (SIZE(0.. maxnoofQoSFlows)) OF UL-QoS-Flows-NotMapped-To-DRB-Item

UL-QoS-Flows-NotMapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIndicator							QoSFlowIndicator,

iE-Extensions				ProtocolExtensionContainer { { UL-QoS-Flows-NotMapped-To-DRB-ItemExtIEs} } OPTIONAL

}

UL-QoS-Flows-NotMapped-To-DRB-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

AsymmetricUL-Flows-MappedToDRB-Item	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF AsymmetricUL-Flows-MappedToDRB-Item-Item

AsymmetricUL-Flows-MappedToDRB-Item-Item 	::= SEQUENCE {

qoSFlowIndicator							QoSFlowIndicator,

uL-qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions				ProtocolExtensionContainer { { AsymmetricUL-Flows-MappedToDRB-ItemExtIEs} } OPTIONAL

}

AsymmetricUL-Flows-MappedToDRB-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

------------------------

Skip to next change

------------------------

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-UL-Count								PDCP-Count								OPTIONAL,

pDCP-DL-Count								PDCP-Count								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AsymmetricUL-QoS-Information		CRITICALITY ignore	EXTENSION AsymmetricUL-QoS-Information		PRESENCE optional },

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-UL-Count								PDCP-Count							OPTIONAL,

pDCP-DL-Count								PDCP-Count							OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AsymmetricUL-QoS-Information		CRITICALITY ignore	EXTENSION AsymmetricUL-QoS-Information		PRESENCE optional },

...

}

------------------------

Skip to next change

------------------------

### 9.4.7	Constant Definitions

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

------------------------

Skip to next change

------------------------

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-AsymmetricUL-QoS-Information									ProtocolIE-ID ::= xx

END