3GPP TSG-RAN WG3 Meeting #101bis	R3-185873

# Chengdu, P.R. China, 8th- 12th October  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDU Session Split at UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction on the supporting of PDU session split at UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU session split at UPF are added to the related Chapters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDU session split at UPF is not described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.4.x, 16.3.4.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 16.3.4.x	PDU Session Split at UPF during PDU session resource setup

When new PDU sessions need to be established with two NG-U tunnels, the 5GC provides two UL TEID addresses during PDU Session Resouce Setup, to be applied as the first  UL tunnel on the NG-U interface and the additional NG-U tunnel. The MN performs SN Addition. The MN provides an DL TEID addresses to be applied as the first and the additional DL tunnel address on the NG-U interface. MN also provide which QoS flows are associated with which tunnel.

Figure 16.3.4.x-1: PDU Session Split at UPF during PDU session resource setup

#### 16.3.4.y	PDU Session Split at UPF during PDU session resource modify

When PDU sessions resource is modified and need to be established with two NG-U tunnels, the 5GC provides an additional UL TEID addresses during PDU Session Resouce Modify Request, to be applied as the additional NG-U tunnel. The MN may perform SN Addition or SN modification. The MN provides an DL TEID addresses to be applied as the additional DL tunnel address on the NG-U interface. MN also provide which QoS flows are associated with which tunnels.

Figure 16.3.4.y-1: PDU Session Split at UPF during PDU session resource modify