3GPP TSG-RAN WG3 #101bis	R3-185878

Chengdu, China, 8th – 12th October 2018

Agenda Item:	21.2.1

Source:	Ericsson

Title:	Discussion on support for Resource-Efficient PDCP Duplication

Document for:	Discussion, Decision

# Introduction

In RAN#81 the revised study item on NR Industrial Internet of Things (NR-IIoT, [1]) has been approved. One of the study areas is this SI is “L2/L3 enhancements”. This contribution focus on the sub-area “resource efficient PDCP duplication”.

# Discussion

Duplication of packet is an expensive feature from a resource point of view. Thus, if the requirements for a certain IIoT application can be met without using duplication, it may be beneficial to deactivate PDCP data duplication. Likewise, when the network evaluates that the requirements might not be met without PDCP data duplication, the network should be capable to activate PDCP data duplication or perform other necessary actions to be able to meet the given requirements.

RAN2 and RAN3 specifications provides a number of tools to allow enable an efficient usage of PDCP data duplication:

- Dynamic (de-)activation of PDCP data duplication
PDCP data duplication is configured by RRC. RRC may also provide the initial activation status. However, both the MCG and SCG may further activate or deactivate duplication by means of MAC CEs.
MAC CEs allow a much faster, dynamic, and timely manner option to activate and deactivate duplication which enables a higher resource efficiency. 

Activation and deactivation of PDCP data duplication is a network decision and decisions could be based on different measurements performed at the network. These mechanisms (measurements and triggers) are not specified and, when available, they are specific to the network implementation. 
It could be discussed if there are any valuable measurements made by the UE which add additional value on top of what the network could do.

- Discarding a duplicate PDUs when one PDU is received
When the PDCP entity receives an indication from lower layers than a certain PDCP PDU has been received by the other end, the PDCP entity indicates to the second RLC entity to discard the corresponding duplicated PDCP PDU. It is supported to “flush” all the PDCP PDUs in the lower layer up to the given PDCP PDU SN, or to discard multiple blocks by specifying how many blocks and the start and the end of the PDCP PDU SN.
This enables an efficient utilization of the network resources.

There may be other areas of improvements to improve the resource efficiency in the network:

- Smart PDCP data duplication
In this context and given that the PDCP data duplication is allowed, smart PDCP data duplication refers to two aspects: 
1) the PDCP entity decides whether to duplicate or not duplicate each of the packets (per packet duplication) – currently, when PDCP data duplication is activated, it applies to all packets. However, a more efficient method to do that would be if the PDCP entity can decide on per packet basis if duplication is needed or not.

2) the PDCP (and even RLC) entity decides whether a packet or a duplicated packet can be discarded at a given time for example if the packet does not meet the delay budget requirements. 
There may be cases in which PDCP PDUs or RLC SDUs/PDUs are buffered for too long in the queues which could lead packet to not meet the latency requirements, for instance. Thus, removing such packets could be beneficial to increase the resource efficiency as those packets might not longer be needed by the receiver side.

RAN3 to discuss and outline the areas in which valuable resource efficiency improvements may be achieved e.g. discarding PDCP and SDU/PDUs

# Conclusion

Based on the discussion above, we propose the following:

Proposal 1	RAN3 to discuss and outline the areas in which valuable resource efficiency improvements may be achieved e.g. discarding PDCP and SDU/PDUs

# References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018