3GPP TSG-RAN WG3#101bis	R3-185889

# Chengdu, P.R. China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | XXXX | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ANR related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To Clarify that the UE may report noSIB1 indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on the UE report of NR cells when there is no SIB1 associated to the detected cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not correctly reflect the ANR functionality developments in 3GPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ************************************* Beginning of changes *********************************************************

#### 15.3.3.2	Intra-system – intra NR Automatic Neighbour Cell Relation Function

ANR relies on NCGI (see subclause 8.2).

Figure 15.3.3.2-1: Automatic Neighbour Relation Function

Figure 15.3.3.2-1 depicts an example where the gNB serving cell A has an ANR function. In RRC_CONNECTED, the gNB instructs each UE to perform measurements on neighbour cells. The gNB may use different policies for instructing the UE to do measurements, and when to report them to the gNB. This measurement procedure is as specified in TS 38.331[12].

1.	The UE sends a measurement report regarding cell B. This report contains Cell B's PCI, but not its NCGI.

When the gNB receives a UE measurement report containing the PCI, the following sequence may be used.

2.	The gNB instructs the UE, using the newly discovered PCI as parameter, to read the all the broadcast NCGI(s), TAC(s), RANAC(s), PLMN ID(s) and NR frequency band(s) of the related neighbour cell. To do so, the gNB may need to schedule appropriate idle periods to allow the UE to read the NCGI from the broadcast channel of the detected neighbour cell. How the UE reads the NCGI is specified in TS 38.331.

3.	When the UE has found out the new cell's NCGI(s), the UE reports all the broadcast NCGI(s) to the serving cell gNB. In addition, the UE reports all the tracking area code(s), RANAC(s), PLMN IDs and NR frequency band(s) that have been read by the UE. In case the detected cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331.

4.	The gNB decides to add this neighbour relation, and can use PCI and NCGI(s) to:

a.	Lookup a transport layer address to the new gNB.

b.	Update the Neighbour Cell Relation List.

c.	If needed, setup a new Xn interface towards this gNB.

#### 15.3.3.3	Intra-system – intra E-UTRA Automatic Neighbour Cell Relation Function

Void.

#### 15.3.3.4	Intra-system – inter RAT Automatic Neighbour Cell Relation Function

Void.

#### 15.3.3.5	Inter-system Automatic Neighbour Cell Relation Function

For Inter-RAT ANR, each cell contains an Inter Frequency Search list. This list contains all frequencies that shall be searched.

Figure 15.3.3.5-1: Automatic Neighbour Relation Function in case of E-UTRAN detected cell

Figure 15.3.3.5-1 depicts an example where the gNB serving cell A has an ANR function. In RRC_CONNECTED, the gNB can instruct a UE to perform measurements and detect cells on other RATs. The gNB may use different policies for instructing the UE to do measurements, and when to report them to the gNB.

1	The gNB instructs a UE to look for neighbour cells in the target RATs. To do so the gNB may need to schedule appropriate idle periods to allow the UE to scan all cells in the target RATs.

2	The UE reports the PCI and carrier frequency of the detected cells in the target RATs.

When the gNB receives UE reports containing PCIs of cell(s) the following sequence may be used.

3	The gNB instructs the UE, using the newly discovered PCI as parameter, to read the ECGI, the TAC and all available PLMN ID(s) of the newly detected cell in case of E-UTRA detected cells. The UE ignores transmissions from the serving cell while finding the requested information transmitted in the broadcast channel of the detected inter-system/inter-frequency neighbour cell. To do so, the gNB may need to schedule appropriate idle periods to allow the UE to read the requested information from the broadcast channel of the detected inter-RAT neighbour cell.

4	After the UE has read the requested information in the new cell, it reports the detected ECGI, TAC, and available PLMN ID(s) to the serving cell gNB.

5	The gNB updates its inter-RAT NCRT.

- ************************************* End of changes *********************************************************