3GPP TSG-RAN WG3 Meeting #101bis	R3-186017

Chengdu, China, Oct. 8th – 12th, 2018

Agenda item:		10.8.2.3

Source:	LG Electronics Inc.

Title:	(TP for NR BL CR for TS 37.340) DRB ID Allocation

Document for:		Approval

1. Introduction

In last meeting, we have agreed the principle of DRB ID allocation. This paper is to capture the agreements in the stage 2 specification.

2. Discussion

On DRB allocation, the following agreements were achieved in last meeting:

1) MN sends a DRB ID list in SN add req/mod. Typically, the MN should assign a sufficiently large DRB ID list (i.e. usable for the lifetime of the UE context)

2a) For MN-terminated bearers, no problem;

2b) For SN-terminated bearers, SN allocates DRB ID(s) from the list provided

We should not fail the procedure because of a failure to allocate DRB ID(s) at the SN

The agreement above is the principle on DRB ID allocation, which should be captured in the QoS section of TS 37.340.

Proposal): To adopt the Stage 2 change for TS 37.340

3. Conclusion

In this contribution, the TP on DRB ID allocation was investigated. The following proposal is suggested to RAN3:

Proposal): To adopt the Stage 2 change for TS 37.340

4. References

- RAN3 Chairman note of Meeting #101

Text proposal for TS 37.340

**************************** First change, the unchange text is skipped********************************

8.1	QoS aspects

In EN-DC, the E-UTRAN QoS framework defined in TS 36.300 [2] applies:

-	An S1-U bearer is established between the EPC and the SN for SN terminated bearers;

-	An X2-U bearer is established between the MN and the SN for split bearers, MN terminated SCG bearers and SN terminated MCG bearers;

-	MN terminated and SN terminated bearers may have either MCG or SCG radio resources or both, MCG and SCG radio resources, established;

-	The MN decides the DL UE AMBR and UL UE AMBR limits to be assigned to the SN, and indicates these to the SN:

-	The PDCP entity at the SN applies the received DL UE AMBR limit to the set of all bearers for which the SN hosts PDCP for the UE;

-	The MAC entity at the SN applies the received UL UE AMBR limit to the scheduled uplink radio traffic at the SN for the UE.In MR-DC with 5GC:

-	The NG-RAN QoS framework defined in TS 38.300 [3] applies;

-	QoS flows belonging to the same PDU session may be mapped to different bearer types (see subclause 4.2.2) and as a result there may be two different SDAP entities for the same PDU session: one at the MN and another one at the SN, in which case the MN decides which QoS flows are assigned to the SDAP entity in the SN. If the SN decides that its SDAP entity cannot host a given QoS flow any longer, the SN informs the MN and the MN cannot reject the request;

-	The MN or SN node that hosts the SDAP entity for a given QoS flow decides how to map the QoS flow to DRBs;

-	If the SDAP entity for a given QoS flow is hosted by the MN and the MN decides that SCG resources are to be configured it provides to the SN

-	DRB QoS flow level QoS parameters, which the SN may reject, and

-	QoS flow to DRB mapping information and the respective per QoS flow information;

-	If the SDAP entity for a given QoS flow is hosted by the SN and the SN configures MCG resources, based on offered MCG resource information from the MN, the SN provides to the MN

-	DRB QoS flow level QoS parameters, which the MN may reject, and

-	QoS flow to DRB mapping information and the respective per QoS flow information.

-	If the SDAP entity for a given QoS flow is hosted by the SN, the MN provides sufficient QoS related information to enable the SN to configure appropriate SCG resources and to request the configuration of appropriate MCG resources. The MN may offer MCG resources to the SN and may indicate for GBR QoS flows the amount offered to the SN on a per QoS flow level.

-	MN decides the PDU session AMBR limits to be assigned to the SN, and indicates these to the SN.

To support PDU sessions mapped to different bearer types, MR-DC with 5GC provides the possibility for the MN to request the 5GC:

-	For some PDU sessions of a UE: Direct the User Plane traffic of the whole PDU session either to the MN or to the SN. In that case, there is a single NG-U tunnel termination at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.

-	For some other PDU sessions of a UE: Direct the User Plane traffic of a subset of the QoS flows of the PDU session to the SN (respectively MN) while the rest of the QoS flows of the PDU session is directed to the MN (respectively SN). In that case, there are two NG-U tunnel terminations at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.

To support notification control indication for GBR QoS flows along the QoS framework specified in 38.300 [3] for MR-DC with 5GC, SN and MN may mutually indicate whenever QoS requirements for GBR QoS flows cannot be fulfilled anymore or can be fulfilled again.

To support the DRB ID allocation for MR-DC with 5GC, MN assigns a sufficiently large DRB ID list, i.e. usable for the lifetime of the UE context. MN sends the list to SN by SN Addition procedure or SN Modification procedure. For SN-terminated bearers, SN allocates DRB ID(s) from the list it received.

*************************************** end of change ********************************************