3GPP TSG-RAN WG3 Meeting #101bis	     R3-186065

Chengdu, P.R. China, 8th - 12th October 2018

Agenda Item:	31.3.4.12

Source:	Nokia, Nokia Shanghai Bell

Title:	Response to R3-185804

Document for:	Discussion and approval

# Introduction

Tdoc R3-185804 deals with the NG Release procedure and includes the list of Active PDU Session IDs in the NG Release Request message. This list is assumed necessary for the AMF to be able to trigger the Deactivate PDU Session corresponding to the involved active PDU sessions. It is claimed that the AMF may not have the information when the feature “decoupling between store and compute” is used because then the UE context (which includes the list of active PDU sessions) maybe located in the UDSF and not available at the AMF.

At last RAN3#101 RAN3 agreed for similar reasons already a CR adding this list of Active PDU Session IDs into the NG Release Complete without deep checking.

# Discussion and proposal

We would like in this paper to re-think the whole issue from a different perspective.

We think the issue is independent from the “compute and storage” and where the UE context is.

Let us take the dual example of a Service Request incoming to the AMF. The UE sends a bitmap of services it wants active/inactive. AMF needs two pieces of information from the UE context:

- Compare with the current list of active PDU sessions,

- Retrieve the relevant involved SMF(s) to trigger the activation.

Either the contacted AMF has the information in the UE context and can proceed. Or, if the UE context is in UDSF, the contacted AMF needs to retrieve the information from the UDSF and proceed.

The N2 release procedure needs the same type of information for AMF to proceed with the deactivation of released PDU session resources:

- AMF needs the current list of active PDU sessions,

- AMF needs to retrieve which SMF(s) are in charge of these PDU sessions in order to trigger the deactivation of these PDU sessions.

As a consequence, there are two cases for AMF when receiving the NG Release Complete (resp. Request):

- Either the AMF has the UE context which contains already the list of Active PDU Session IDs,

- Or the AMF does not have the UE context and NG-RAN providing the list of Active PDU Session IDs in the NG Release message will not help because the AMF will anyway need to retrieve the full UE context from the UDSF in order to determine the list of involved SMFs.

Observation 1: NG-RAN node providing the list of Active PDU Session IDs in NG Release Request (resp. complete) message to the AMF seems useless.

Even though not useful, providing the list of Active PDU Session IDs to the AMF is on the other hand not harmful, just redundant information. Unless AMF starts to do some consistency checks.

NOTE: providing the list of Active PDU Session IDs to the AMF could be useful if NG-RAN node also provides the list of involved SMFs, however this is obviously not a possible option.

Given that RAN3 is currently “half-way” i.e. has agreed a CR for NG Release Complete while not yet for NG Release Request, we would like to propose two possible way forward(s):

Way forward 1: agree Huawei TP in R3-185804 introducing at this meeting the list of Active PDU Session IDs in the NG Release Request message to be consistent with last RAN3 CR to include it in the NG Release Complete, even if the information seems useless.

Way forward 2: remove the list of Active PDU Session IDs in NG Release Complete message and liaise SA2 about our findings.

If RAN3 decides to go for the way forward 2 the TP is provided in the annex.

Proposal 1: RAN3 to double-check and decide between way forward 1 or way forward 2.

# Annex: TP TS 38.413 (case RAN3 decides way forward 2)

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<Unchange Text Omitted>

### 9.4.4	PDU Definitions

<Unchange Text Omitted>

PDUSessionResourceHandoverList,

PDUSessionResourceListHORqd,

<Unchange Text Omitted>

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceListHORqd,

<Unchange Text Omitted>

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseComplete-IEs} },

...

}

UEContextReleaseComplete-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging	PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

<Unchange Text Omitted>

### 9.4.5	Information Element Definitions

<Unchange Text Omitted>

<Unchange Text Omitted>

### 9.4.7	Constant Definitions

<Unchange Text Omitted>

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61