3GPP TSG-RAN WG3 Meeting #101bis 	     		R3-186095

Chengdu, China, 8th - 12th October 2018

Source: 	AT&T

Title:	Performance results for hop-by-hop vs. end-to-end RLC ARQ

Agenda Item:	24.1.2.1

Document for:	Discussion

# Introduction

For IAB architecture 1a, as described in the draft IAB SI report TR 38.874 [1], currently two different modes of operation for multi-hop RLC ARQ are being studied - hop-by-hop RLC ARQ and end-to-end RLC ARQ. A couple of example user plane protocol stacks depicting these RLC ARQ modes are shown below in Figure 1 for illustration purposes.

Figure 1: Example user plane protocol stack with a) end-to-end RLC ARQ and b) hop-by-hop RLC ARQ

There has been much debate in RAN2 about the pros and cons of these RLC ARQ configurations. A table summarizing some of these points has been captured in the IAB SI TR draft [1] as well. However, so far, no company has provided performance results comparing the two modes of operation in an FR2-based NR IAB deployment scenario.

In this contribution we provide simulation results comparing the performance of end-to-end vs. hop-by-hop RLC ARQ using a sophisticated NR system level simulator that models physical layer and higher layer protocol details, including mmWave channel model with blockage. Section 2 summarizes the system level simulation model assumptions and provides performance results, and Section 3 provides conclusions.

# System level simulation results comparing performance of end-to-end vs. hop-by-hop RLC ARQ

A detailed dynamic system level simulator is used to compare user throughput performance results of end-to-end vs. hop-by-hop RLC ARQ in a multi-hop IAB deployment for the homogeneous scenario. System level simulation results follow agreed system level evaluation assumptions from Table A.1.1-1 in Appendix A of TR 38.874 [1].  A few of the modeled assumptions are summarized below:

- 30GHz carrier frequency, 400MHz BW

- Homogeneous Scenario: 3 Donors, 18 IAB-nodes, 105 UEs

- 1Mb file size, TCP segmentation ON

- Average blockage duration = 100ms [See 38.901 for details]

Additional comments regarding the system level simulations:

- For the end-to-end RLC ARQ case, since the intermediate IAB nodes perform RLC segmentation/reassembly functions while the RLC AM entities reside at the donor DU and UE, a new message called RLC_Reassembly_Failure had to be introduced to allow intermediate IAB nodes to convey a failed PDCP PDU reassembly attempt over an IAB hop to the destination RLC AM entity. Note that this new message may need to travel across more than one hop to reach the destination RLC AM entity.

- The provided simulation results may be slightly optimistic for the end-to-end RLC ARQ case, because the transmission of RLC status reports from the destination RLC AM entity to the source RLC AM was assumed to be lossless.

Figure 1: End-to-End Packet Throughput (Medium and High Load Cases)

Figure 2: Number of RLC segmentation entities traversed by RLC SDUs

Figure 1 shows CDFs of end-to-end perceived packet throughput for medium and high load cases. As can be observed from the results, especially for the medium load case, the hop-by-hop RLC ARQ case experiences an almost 50% better 10th percentile and median packet throughput compared to the end-to-end RLC ARQ case. For the high load case, the gain of hop-by-hop RLC ARQ over end-to-end RLC ARQ is a bit more modest at around 8%.

Figure 2 shows CDFs of number of RLC segmentation entities traversed by RLC SDUs. This is a good indication of how much an RLC SDU had to travel before being successfully received at the PDCP layer at the UE. This can be explained better with an example. Assume that the route traversed by an RLC SDU is donor DU  IAB Node 1  IAB Node 2  UE. For the hop-by-hop RLC ARQ case, if the SDU experienced one RLC retransmission over the IAB Node 1  IAB Node 2 link, then the RLC segmentation entities traversed by the SDU would be Donor DU  IAB Node 1  IAB Node 1 (retx)  IAB Node 2 (So a total of 4). Keep in mind that we are counting RLC segmentation entities, not RLC reassembly entities. For the end-to-end RLC ARQ case, if the SDU experienced one RLC retransmission, the RLC segmentation entities traversed by the SDU would be Donor DU  IAB Node 1  IAB Node 2  IAB Node 1 (retx)  IAB Node 2 (retx) (So a total of 5). The results indicate that with end-to-end RLC ARQ, the RLC SDUs need to travel through more RLC segmentation entities, thereby negatively impacting end-to-end perceived packet throughput.

Observation 1: In moderate to high loaded IAB networks, system level simulations show that hop-by-hop RLC ARQ provides approximately 8%-48% better 10%-ile and median packet throughput compared to the end-to-end RLC ARQ case in an FR2 deployment with blockage.

# Conclusion

In this contribution we provided simulation results comparing the performance of end-to-end vs. hop-by-hop RLC ARQ using a sophisticated NR system level simulator that models physical layer and higher layer protocol details, including mmWave channel model with blockage. The following observation was made:

Observation 1: In moderate to high loaded IAB networks, system level simulations show that hop-by-hop RLC ARQ provides approximately 8%-48% better 10%-ile and median packet throughput compared to the end-to-end RLC ARQ case in an FR2 deployment with blockage.

# References

- Draft TR 38.874 v0.4, Study on Integrated Access and Backhaul.