3GPP TSG-RAN WG3 #101bis	   R3-186140

Chengdu, China, 8th October – 12th October 2018

Source: Qualcomm Incorporated

Title: (TP for MR-DC with 5GC BL CR for TS 37.340) MN DL TNL address delivery for flow level offload in MR-DC with 5GC

Document for: Approval

Agenda Item:

# Introduction

This paper provides TP for MN TNL address delivery in SN Addition/Modification Preparation procedures, based on R3-185513.

# TS 37.340: Text Proposal

----------------------- Start of TP--------------------------------------------

## 10.2	Secondary Node Addition

### 10.2.2	MR-DC with 5GC

Editor's note: MR-DC with the 5GC is not complete and is targeted for completion in December 2018.

The Secondary Node (SN) Addition procedure is initiated by the MN and is used to establish a UE context at the SN in order to provide radio resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the initial SCG serving cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). Figure 10.2.2-1 shows the SN Addition procedure.

Figure 10.2.2-1: SN Addition procedure

1.	The MN decides to request the target SN to allocate radio resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info). In addition, for bearers requiring SCG radio resources, MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. The MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision. MN needs to provide Xn-U UL TNL address information for MN terminated bearers. The SN may reject the request.

NOTE 1:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 2:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources. For bearers requiring SCG radio resources the SN triggers UE Random Access so that synchronisation of the SN radio resource configuration can be performed. The SN decides for the PScell and other SCG Scells and provides the new SCG radio resource configuration to the MN in a SN RRC configuration message contained in the SN Addition Request Acknowledge message. In case of bearer options that require Xn-U resources between the MN and the SN, the SN provides Xn-U TNL address information for the respective E-RAB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers.  For SN terminated bearers, the SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

2a. In order to enable data forwarding for the bearers that have been modified to SN terminated bearers requiring MCG resources, the MN sends the Data Forwarding Address Indication message including the Xn-U DL TNL information, immediately after receiving step 2.

NOTE 3:	In case of MN terminated bearers, transmission of user plane data may take place after step 2.

NOTE 4:	In case of SN terminated bearers, data forwarding and the SN Status Transfer may take place after step 2.

3.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message, without modifying it.

4.	The UE applies the new configuration and replies to MN with MN RRC reconfiguration complete message, including a SN RRC response message for SN, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the encoded SN RRC response message, if received from the UE.

6.	If configured with bearers requiring SCG radio resources, the UE performs synchronisation towards the PSCell configured by the SN. The order the UE sends the MN RRC reconfiguration complete message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	In case of SN terminated bearers using RLC AM, the MN sends SN Status Transfer.

8.	In case of SN terminated bearers using RLC AM, and dependent on the bearer characteristics of the respective QoS Flows, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

9-12.	For SN terminated bearers, the update of the UP path towards the 5GC is performed via PDU Session Path Update procedure.

## 10.3	Secondary Node Modification (MN/SN initiated)

### 10.3.2	MR-DC with 5GC

Editor's note: MR-DC with the 5GC is not complete and is targeted for completion in December 2018.

The SN Modification procedure may be initiated either by the MN or by the SN and be used to modify, establish or release PDU session/QoS Flow contexts, to transfer PDU session/QoS Flow contexts to and from the SN or to modify other properties of the UE context within the same SN. It may also be used to transfer an NR RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used).

The SN modification procedure does not necessarily need to involve signalling towards the UE.

MN initiated SN Modification

Figure 10.3.2-1: SN Modification procedure - MN initiated

The MN uses the procedure to initiate configuration changes of the SCG within the same SN, including addition, modification or release PDU session/QoS Flows mapped onto SN terminated bearer(s) and MN terminated bearers with an SCG RLC bearer. The MN uses the procedure to query the current SCG configuration, e.g. when delta configuration is applied in a MN initiated SN change. MN may not use the procedure to initiate the addition, modification or release of SCG Scells. The SN may reject the request, except if it concerns the release of PDU session/QoS flow. Figure 10.3.2-1 shows an example signalling flow for a MN initiated SN Modification procedure.

1.	The MN sends the SN Modification Request message, which may contain PDU session/QoS Flow context related or other UE context related information, data forwarding address information (if applicable), PDU session level Network Slice info and the requested SCG configuration information, including the UE capabilities coordination result to be used as basis for the reconfiguration by the SN.

2.	The SN responds with the SN Modification Request Acknowledge message, which may contain new SCG radio configuration information within a SN RRC configuration message, and data forwarding address information (if applicable).

2a. In order to enable data forwarding for the bearers that have been modified to SN terminated bearers requiring MCG resources, the MN sends the Data Forwarding Address Indication message including the Xn-U DL TNL information, immediately after receiving step 2.

3/4.	The MN initiates the RRC connection reconfiguration procedure, including SN RRC configuration message. The UE applies the new configuration and replies with MN RRC reconfiguration complete message, including a SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Reconfiguration Complete message.

6.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

7.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status transfer.

8.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-1 depicts the case where a PDU session/QoS Flow context is transferred from the MN to the SN).

9.	If applicable, a PDU Session path update procedure is performed.