3GPP TSG-RAN WG3#101bis                                                                       R3-186170

Chengdu, China, 8-12 Oct 2018

Agenda Item:	31.3.4.8

Source:	CATT

Title:	Summary of the offline discussion on UAC barring information

Document for:	Discussion, Decision

1. Introduction

CB: # 53_ACinfo

- DU is in charge of radio so should have the final say - clarify

- possible enhancements of info from CU to DU?

-  check details

(CATT)

Issue 1: Which node is in charge of the final decision of UAC barring information?

During the online discussion, it is assumed that DU should perform the final decision of UAC barring information since DU is in charge of the radio. However, there are some doubts from some company on this assumption. The reason is that CU has the overall view of the status of all DU it connected.

For both of the camp, the common part is that both CU and DU needs to be involved in the configuration of UAC barring information. So, information needs to be transferred between CU and DU. Since we have agreed that SIB1 is encoded by DU, some companies think DU could make the final decision.

Conclusion : Based on the agreement that SIB1 is encoded by DU, some companies think that DU makes the final decision for UAC Barring information. However, some companies have different opinion on it and think CU should make the final decision.

Issue 2: Information transferred from CU to DU

For the information transferred from CU to DU, different alternatives are proposed during the online session

Alternative 1: The information transferred from CU to DU refer to the definition of UAC Barring information defined in RAN2

Alternative 2: CU provides some assistance information to DU to help DU update the UAC barring information e.g. barring factor/barring time for each UE category.

Alternative 3: CU conveys portions of SIB1 (including use case for uac-barringInfo)to DU which means all UAC information defined in SIB1 is transferred and can adapt possible future extension in RAN2 without impact of RAN3 specification..

Proposal: It is proposed to discuss the above three  alternatives and select one in next RAN3 meeting.