3GPP TSG-RAN WG3 Meeting #101bis	R3-186181

# Chengdu, P.R. China, 8th- 12th October  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDU Session Split at UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction on the supporting of PDU session split at UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU session split at UPF are added to the related Chapters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDU session split at UPF is not described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.x, 10.x.x, 10.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.x	PDU Session Split at UPF

### 10.x.x	PDU Session Split at UPF during PDU session resource setup

When a new PDU session needs to be established with two NG-U tunnels, the 5GC provides two UL TEID addresses during PDU Session Resouce Setup. The MN provides two DL TEID addresses and also the QoS flows associated with the tunnel.

Figure 10.x.x-1: PDU Session Split at UPF during PDU session resource setup

1.	The 5GC provides two UL TEID addresses during PDU Session Resouce Setup, to be applied as the first UL tunnel on the NG-U interface and the additional NG-U tunnel.

2.	The MN decides to setup two tunnels/PDU sessions. MN uses the seconday Node Addition procedure or seconday node modification procedure.

3.	The MN sets up a DRBs.

4.	UE establishs the DRBs and sends back DRB setup complete.

5.	The MN provides an DL TEID addresses to be applied as the first and an additional DL tunnel address on the NG-U interface. MN also provide which QoS flows are associated with which tunnel.

### 10.x.y	PDU Session Split at UPF during PDU session resource modify (5GC initiated)

When PDU sessions resource is modified to be split into two NG-U tunnels, the 5GC provides an additional UL TEID addresses during PDU Session Resouce Modify Request. The MN may perform SN Addition or SN modification. The MN provides a DL TEID addresses to be applied as the additional DL tunnel address and the QoS flows associated to the tunnel.

Figure 10.x.y-1: PDU Session Split at UPF during PDU session resource modify

1.	The 5GC provides an additional UL TEID addresses during PDU Session Resouce Modify Request, to be applied as the additional NG-U tunnel.

2.	The MN decides to setup two tunnels/PDU session. If the new tunnel is to be setup at SN, the MN may perform SN Addition as described in 10.2.2 up to step 6 or the MN performs SN modification as described in 10.3.2 up to step 8.

3.	The MN provides an DL TEID addresses to be applied as the additional DL tunnel address on the NG-U interface and the QoS flows associated with it.