3GPP TSG RAN WG3 Meeting #101bis	R3-186203

Chengdu/China, 8-12th October 2018

Title:	NTN mobility management principles

Source: 	Thales

Type:	Discussion

Document for:	Agreement

Agenda Item:	20.2 Architecture

Study item:	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Abstract

This technical contribution aims at providing proposals on mobility management aspects related to Non-Terrestrial network (study item in RP-181598 Study on solutions for NR to support non-terrestrial network, Thales).

# Mobility management aspects

## NTN cell and beam definitions

These following definitions (NTN cell and NTN beam) should be confirmed in RAN2 or RAN1

A NTN beam is defined as a beam generated by satellite / HAPS.

A NTN cell is defined as “NTN beam foot print on earth”

- For scenarios A, B, C2 & D2, The NTN cell corresponds to the beam foot print. It may be fixed on earth in the case of GEO or moving on earth in the case of LEO.

- For scenario C1 & D1 (LEO generating fixed beam foot print on earth), the NTN cell corresponds to the minimum foot print on earth of beams from the serving satellites”

- Within RAN3, the focus will be on studying the impacts associated to earth fixed and earth moving NTN cells

## Idle mode mobility

Proposal 2.2.1: it is recommended to adopt tracking areas fixed on earth.

Note: This is implemented in all operational Non-GEO system (whatever earth fixed and moving beam foot print)

Proposal 2.2.2: A registration area encompass one or several tracking areas.

Proposal 2.2.3: A registration area may be bounded by a border between two countries.

Proposal 2.2.4: A NTN cell (foot print of one beam) may encompass several tracking areas

Proposal 2.2.5: A UE may have the capability to determine its location (e.g. GNSS)

Proposal 2.2.6: The network may have the capability to determine the UE location

Proposal 2.2.7: The paging procedure described in R3-185700 (Nokia) can be considered as one possible solution. It assumes that UE is able to determine its position

Proposal 2.2.8: Each NTN beam should be assigned a specific PCI. The max number of PCI (currently 1008) may have to be extended to prevent PCI conflict especially between satellite of different orbit planes

Note:  For LEO generating fixed beam foot print on earth, an identifier may also be assigned to the NTN cell. To be further studied if needed

## Connected mode mobility

Proposal 2.3.1: There are different type of hand-over in Non-Terrestrial networks

- Inter beam hand-over (between beam generated by same satellite)

- Inter satellite hand-over (between beam generated by different satellite)

- Inter access hand-over (between cellular and satellite access)

Proposal 2.3.2: A tentative identification of applicable 3GPP hand-over procedures to support the different types of NTN hand-over is provided in the table below:

| NTN hand-over type/ scenarios | Scen A. Transparent GEO | Scen B. Regenerative GEO | Scen C1. Transparent LEO beam foot print fixed on earth | Scen C2. Transparent LEO beam foot print moving on earth | Scen D1. Regenerative LEO beam foot print fixed on earth | Scen D2. Regenerative LEO beam foot print moving on earth |
| --- | --- | --- | --- | --- | --- | --- |
| Inter beam hand-over | Intra or Inter gNB hand-over | Intra gNB hand-over | Intra or Inter gNB hand-over | Intra gNB hand-over | Inter gNB hand-over | Intra gNB hand-over |
| Inter satellite hand-over | Inter gNB hand-over | Inter gNB hand-over | Intra gNB hand-over | Intra gNB hand-over | Inter gNB hand-over | Inter gNB hand-over |

Proposal 2.3.3: Inter access hand-over (between cellular and satellite access) is considered with Inter gNB procedure via the 5GCN and via the Xn