3GPP TSG-RAN WG3#102                                                                         R3-186551

Spoken, USA, 12-16 Nov 2018

Agenda Item:	31.3.4.9

Source:	CATT

Title:	(TP for NR BL CR for TS 37.340) Correction on PCI confusion

Document for:	Decision

1. Introduction

In RAN3#101 meeting, it is agree to clarify that “Neighbor cell relationship is provided via non-UE associated signaling (e.g. Xn setup procedure, NG RAN node configuration procedure).” for option 4/7.However,this part is missing for option 3,we propose to add the similar sentence for option3.

The TP is provided in the Annex.

2.Annex TP for 37.340

<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>

5	Layer 1 related aspects

In MR-DC, two or more Component Carriers (CCs) may be aggregated over two cell groups. A UE may simultaneously receive or transmit on multiple CCs depending on its capabilities. The maximum number of configured CCs for a UE is 32 for DL and UL. Depending on UE's capabilities, up to 31 CCs can be configured for an E-UTRA cell group when the NR cell group is configured.

In EN-DC, the SN may configure the same Physical Cell ID (PCI) to more than one NR cell it serves. To avoid PCI confusion for EN-DC, NR PCIs should be allocated in a way that an NR cell is uniquely identifyable by E-UTRA cell information. This E-UTRA cell is in the coverage area of an NR cell included in the EN-DC operation. In addition, NR PCIs should only be re-used in NR cells on the same SSB frequency sufficiently distant from each other. Neighbor cell relationship is provided via non-UE associated signaling (e.g. EN-DC X2 setup procedure, EN-DC  configuration update procedure).X2-C signalling supports disambiguation of NR PCIs by including the ECGI in respective X2AP EN-DC messages (e.g. SGNB ADDITION REQUEST).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>