3GPP TSG RAN WG3 Meeting #102 	R3-186729

Spokane/USA, 12-16 November 2018

Title:	NTN hand-over types

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2.4.1

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at identifying the various types of hand-over in Non-Terrestrial networks.

# Discussion

One should distinguish between

- Intra satellite/Inter beam hand-over

- Inter satellite hand-over

- Inter access hand-over between cellular and satellite access

There could be some variants depending on whether

- The satellite is transparent or regenerative

- The hand-over requires a change of gNB or not.

## 2.1 Transparent satellite

In terms of mobility, there are several hand-over types with transparent payload satellites:

- Intra satellite/Inter beam hand-over with same gNB/NTN gateway. This may be achieved with a channel re selection procedure

- Intra satellite/Inter beam hand-over with different gNB/NTN gateway.  This may apply with GEO satellite served by a set of gNB/NTN gateways distributed in the satellite coverage. Two adjacent beams may be served by two different gNB/NTN gateways. This requires an inter gNB procedure

- Inter satellite hand-over with same gNB/NTN Gateway. This applies typically with LEO satellites that successfully serve a give UE. This requires an intra gNB procedure

- Inter satellite hand-over with different gNB/NTN Gateway. This may apply with LEO satellites that successfully serve a give UE. This requires an inter gNB procedure

- Inter access hand-over with same gNB. This requires an intra gNB procedure

- Inter access hand-over with different gNB. This requires an inter gNB procedure

Figure 1.1: Intra satellite/Inter beam hand-over with same gNB/NTN gateway (Transparent satellite)

Figure 1.2: Intra satellite/Inter beam hand-over with different gNB/NTN gateway (Transparent satellite)

Figure 1.3: Inter satellite hand-over with same gNB/NTN gateway (transparent satellite)

Figure 1.4: Inter satellite hand-over with different gNB/NTN gateway (transparent satellite)

Figure 1.5: Inter access hand-over with same gNB (transparent satellite)

Figure 1.6: Inter access hand-over with different gNB (transparent satellite)

Proposal 1: To consider in the study the following types of hand-over involving Transparent satellite access

- Intra satellite/Inter beam hand-over with same or different gNB.

- Inter satellite hand-over with same or different gNB.

- Inter access hand-over with same or different gNB

## 2.2 gNB based satellite

In terms of mobility, there are three possible hand-over types with gNB based payload satellites:

- Intra satellite/Inter beam hand-over. This may be achieved with an intra gNB hand-over procedure

- Inter satellite hand-over with same or different NTN gateway. This may be achieved with an inter gNB hand-over procedure via Xn over ISL

- Inter access hand-over. This may be achieved with an hand-over procedure via the 5GCN

Figure 2.1: Intra satellite/Inter beam hand-over (gNB based satellite)

Figure 2.2: Inter satellite hand-over with same NTN gateway (gNB based satellite)

Figure 2.3: Inter satellite hand-over with different NTN gateway (gNB based satellite)

Figure 2.4: Inter access hand-over (gNB based satellite)

Proposal 2: To consider in the study the following types of hand-over involving gNB based satellite access:

- Intra satellite/Inter beam hand-over

- Inter satellite hand-over with same or different NTN gateway.

- Inter access hand-over

# Conclusion

Proposal 1: To consider in the study the following types of hand-over involving Transparent satellite access

- Intra satellite/Inter beam hand-over with same or different gNB.

- Inter satellite hand-over with same or different gNB.

- Inter access hand-over with same or different gNB

Proposal 2: To consider in the study the following types of hand-over involving gNB based satellite access:

- Intra satellite/Inter beam hand-over

- Inter satellite hand-over with same or different NTN gateway.

- Inter access hand-over

Proposal 3: The following NG-RAN procedures may apply for the different hand-over types

|  | Transparent satellite | Regenerative satelliye (gNB based) |
| --- | --- | --- |
| Intra satellite/Inter beam hand-over | channel re selection procedure or inter gNB procedure respectively if same or different gNB/NTN gateway involved | intra gNB hand-over procedure |
| Inter satellite hand-over | Intra or inter gNB procedure respectively if same or different gNB/NTN gateway involved | inter gNB hand-over procedure |
| Inter access hand-over | Intra or inter gNB procedure respectively if same or different gNB/NTN gateway involved | hand-over procedure via the 5GCN |

# Text Proposal for TR 38.821 v0.2.0

START OF CHANGES

## 8.2	Connected mode mobility (FFS)

There are different types of hand-overs in Non-Terrestrial networks:

- Intra-satellite hand-over (between beams generated by same satellite)

- Inter-satellite hand-over (between beams generated by different satellite)

- Inter-access hand-over (between cellular and satellite access)

They are illustrated below.

### 8.2.1	Hand-over involving Transparent based satellite access

Figure 8.2.1-1: Intra satellite/Inter beam hand-over with same gNB/NTN gateway (Transparent satellite)

Figure 8.2.1-2: Intra satellite/Inter beam hand-over with different gNB/NTN gateway (Transparent satellite)

Figure 8.2.1-3: Inter satellite hand-over with same gNB/NTN gateway (transparent satellite)

Figure 8.2.1-4: Inter satellite hand-over with different gNB/NTN gateway (transparent satellite)

Figure 8.2.1-5: Inter access hand-over with same gNB (transparent satellite)

Figure 8.2.1-6: Inter access hand-over with different gNB (transparent satellite)

### 8.2.2	Hand-over involving Regenerative based satellite access

Figure 8.2.2-1: Intra satellite/Inter beam hand-over (gNB based satellite)

Figure 8.2.2-2: Inter satellite hand-over with same NTN gateway (gNB based satellite)

Figure 8.2.2-3: Inter satellite hand-over with different NTN gateway (gNB based satellite)

Figure 8.2.2-4: Inter access hand-over (gNB based satellite)

### 8.2.3	Hand-over procedures in NTN

The following NG-RAN procedures applies for the different hand-over types in NTN.

Table 8.2.3-1: NG-RAN procedures versus NTN hand-over types

|  | Transparent satellite | Regenerative satelliye (gNB based) |
| --- | --- | --- |
| Intra satellite/Inter beam hand-over | channel re selection procedure or inter gNB procedure respectively if same or different gNB/NTN gateway involved | intra gNB hand-over procedure |
| Inter satellite hand-over | Intra or inter gNB procedure respectively if same or different gNB/NTN gateway involved | inter gNB hand-over procedure |
| Inter access hand-over | Intra or inter gNB procedure respectively if same or different gNB/NTN gateway involved | hand-over procedure via the 5GCN |

It is FFS the necessary changes to the applicable procedures (Inter or Intra gNB hand-over)

An inter-access hand-over (between cellular and satellite access) is considered by utilizing an inter gNB procedure via the 5GCN (e.g. for Satellite with on board processed payload) or via the Xn (e.g. for satellite with transparent payload).

It is assumed that not all UEs are capable of positioning.

END OF CHANGES