3GPP TSG RAN WG3 Meeting #102	R3-187070

Spokane, WA, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0069 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UP retransmission status reporting in the NR UP protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current DDDS retransmission status reporting is imprecise and ambiguous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Clarification of retransmission PDCP SN reporting related the ‘Highest successfully delivered retransmitted NR PDCP Sequence Number’ and the ‘Highest retransmitted NR PDCP Sequence Number’. -Renaming of the two above sequence number from ‘Highest’ to ‘Latest’ -The corresponding discussion is presented in R3-187069. -This is a backwards-compatible change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imprecise and ambiguous retransmission status reporting in DDDS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.2.1, 5.5.2.2, 5.5.3.30, 5.5.3.31, 5.5.3.32, 5.5.3.33. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

## 5.4	Elementary procedures

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane instance making use of the Transfer of Downlink User Data procedure is associated to a single data radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

<<<<<<<<<<<<<<<<Skipped text>>>>>>>>>>>>>>

-------------------------------------------Change 2-------------------------------------------

### 5.4.2	Downlink Data Delivery Status

#### 5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer. The corresponding node may also transfer uplink user data for the concerned data radio bearer to the node hosting the NR PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the successful delivery of DL control data to the corresponding node.

When the corresponding node decides to trigger the feedback for Downlink Data Delivery procedure it shall report as specified in section 5.2:

a)	in case of RLC AM, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

b)	the desired buffer size in bytes for the concerned data radio bearer;

c)	optionally, the desired data rate in bytes associated with a specific data radio bearer configured for the UE;

d)	the NR-U packets that were declared as being "lost" by the corresponding node and have not yet been reported to the node hosting the NR PDCP entity within the DL DATA DELIVERY STATUS frame;

e)	if retransmission NR PDCP PDUs have been delivered to the UE, the NR PDCP PDU sequence number of the latest retransmission NR PDCP PDU successfully delivered in sequence to the UE;

f)	if retransmission NR PDCP PDUs have been transmitted to the lower layers, the NR PDCP Sequence Number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs transmitted to the lower layers.

<<<<<<<<<<<<<<<<Skipped text>>>>>>>>>>>>>>

-------------------------------------------Change 3-------------------------------------------

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

The following shows the respective DL DATA DELIVERY STATUS frame. The Figure shows an example of how a frame is structured when all optional IEs (i.e. those whose presence is indicated by an associated flag) are present.

Absence of such an IE changes the position of all subsequent IEs on octet level.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | Highest Transmitted NR PDCP SN Ind | Highest Delivered NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare |  |  |  | Data rate Ind. | Latest Retransmitted NR PDCP SN Ind | Latest Delivered Retransmitted NR PDCP SN Ind | Cause Report | 1 |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  | 4 |
| Desired Data Rate |  |  |  |  |  |  |  | 0 or 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  | 0 or 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  | 0 or (6* Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| Cause Value |  |  |  |  |  |  |  | 0 or 1 |
| Latest successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| Latest retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

-------------------------------------------Change 4-------------------------------------------

<<<<<<<<<<<<<<<<Skipped text>>>>>>>>>>>>>>

#### 5.5.3.30	Latest Delivered Retransmitted NR PDCP SN Ind

Description: This parameter indicates the presence of latest successfully delivered retransmitted PDCP Sequence Number.

Value range: {0= Latest successfully delivered retransmitted PDCP Sequence Number not present, 1= Latest successfully delivered retransmitted PDCP Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.31	Latest Retransmitted NR PDCP SN Ind

Description: This parameter indicates the presence of latest retransmitted NR PDCP Sequence Number.

Value range: {0= Latest Retransmitted NR PDCP Sequence Number not present, 1= Latest Retransmitted NR PDCP Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.32	Latest successfully delivered retransmitted NR PDCP Sequence Number

Description: This parameter indicates the NR PDCP Sequence Number of the latest retransmission NR PDCP PDU successfully delivered in sequence to the UE.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.33	Latest Retransmitted NR PDCP Sequence Number

Description: This parameter indicates the NR PDCP Sequence Number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs transmitted to the lower layers.

Value range: {0..218-1}.

Field length: 3 octets.

<<<<<<<<<<<<<<<<Skipped text>>>>>>>>>>>>>>

-------------------------------------------End of changes ------------------------------------------