3GPP TSG-RAN WG3#102                                                                       R3-187107

# Spokane, USA, 12 - 16 November 2018

Agenda Item:	31.3.1.37

Source:	CATT

Title:	Summary of the offline discussion on Default Paging DRX

Document for:	Discussion, Decision

1. Introduction

CB: # 25_DefaultDRX

-  clarify scenario: need to support?

- NG impact?

- F1 impact?

(CATT)

Summary of offline disc R3-187107

2. Summary

-  clarify scenario: need to support?

There are different understandings on whether different default paging DRX may be configured to different DUs within one CU. Some companies including operators think it is a valid scenario since different DU may cover different area which different number of UEs are expected. At the same time, the radio resources assigned for paging may also be different for different DUs. Based on that, it is not reasonable to force all DUs within one CU have the same default paging DRX. However, some companies, which also referred to earlier discussion at RAN3101bis meeting on same topic, is that configuration and support for different Default Paging DRX values per DU is not an scenario that needs to be supported and  instead, such situation shall be avoided.The main reason is that in NG interface,only one default paging DRX is reported to AMF

Conclusion：There is no agreement on need to support the scenario in which different DUs within one CU are configured with different default paging DRX. It could be discussion later.

- F1 impact?

During the offline discussion, it is converged that the default paging DRX should be transferred from DU to CU. One possible way is that CU gets the information from the SIB1 container. Introducing a separate F1AP IE i.e. Default Paging DRX is regarded as an optimization which seems low priority in current stage.

Conclusion：Default Paging DRX is configured to DU via OAM. In current stage, CU just gets the information from DU via SIB1 container. No impact to F1AP.

- NG impact?

Since NG impact depends on whether there is agreement on scenario or not, it is not discussed during the offline discussion.

Conclusion: In current stage, no NG impact is foreseen.