3GPP TSG-RAN WG3 #102                                              	R3-187184 (revision of R3-186316)

Spokane, USA, 12th - 16th November 2018

Agenda Item:	24.1.2.3. Setup, Connection Establishment

Source: 	KDDI Corporation

Title:	TP to TR 38.874: Removal of the FFS on IAB Routing update procedure

Document for:	pCR for TR 38.874

Introduction

In this contribution, we propose to change the text related to IAB Routing update procedure, “IAB node’s integration procedure phase 2-1: Routing update” in Section 9.3 “Integration of IAB-node” in the TR, so that we can remove the FFS in this section.

Proposal: Adopt the text change for IAB Routing update procedure in the TR 38.874 described below, so that we can remove the related FFS

Text Proposal to TR 38.874

<<TP start>>

## 9.3	Integration of IAB-node

IAB-node integration has the following phases:

<<*snip*>>

Figure 9.3-2(b). IAB node’s Integration Procedure Phase 2-1

IAB node’s integration procedure phase 2-1: Routing update

For CP alternative 2,

1.	Identifiers and routing information are configured by F1-AP messages. Donor-CU sends the related F1AP message to the newly integrated IAB node2 to assign its identifier. Donor-CU sends another F1AP message to the intermediate IAB node1 to update its routing information.

2.	The adaptation layer in donor-CU and IAB node2 can include the configured identifier and routing information to each adaptation headers which are attached to the BH packets. The adaptation layer in IAB node1 decide the destination of each BH packets based on updated routing information.

For CP alternative 4,

1.	Identifier assignments are done by using IPv6 mechanism. The donor DU sends out ICMPv6 Router Advertisement (RA) to the IAB node2. The IAB node2, when it receives the ICMPv6 RA, generates 1 or more IPv6 addresses. The IAB node2 announces the IP addresses to the donor DU using ICMPv6 Neighbor Solicitation and Neighbor Advertisement. The donor DU creates a mapping between IP addresses of the IAB nodes and establishes the routing information for IAB nodes, and broadcasts the information.

2.	The adaptation layer in donor-CU and IAB node1, IAB node2 can use IP addresses and the configured IP routing information for BH packets forwarding.

For CP alternative 1, 3, and 5,

1.	Identifiers and routing information are configured by adaptation layer. Donor-CU assigns a new identifier to the IAB node2 by sending an adaptation layer command. Donor-CU sends another adaptation layer command to the intermediate IAB node1 to update the routing information.

2.	The adaptation layer in donor-CU and IAB node2 can include the configured identifier to each adaptation headers which are attached to the BH packets. The adaptation layer in IAB node1 decide the destination of each BH packets based on updated routing information.

Figure 9.3-2(b) depicts the procedure for CP alternative 2.

<<TP end>>