3GPP TSG RAN WG3 Meeting #102	R3-186477

Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	22.3.2

Source:	Ericsson

Title:	TP for TR 38.866: The open issues in RIM

Document for:	pCR to TR 38.866

# Introduction

CB#2 Intra-cluster co-ordination

- Capture possible agreement

- possible TP for TR

(E///)

# ANNEX: TP to TR 38.866

-------------------------------------------Start of changes-------------------------------------------

## 6.3.x	Intra-set coordination in RIM

RAN3 analysed the need for intra-set coordination based on the following assumptions:

- Victim gNB sets: A TDD network is synchronized, where RIM reference signals (RIM-RS) are sent at predefined symbol positions, as configured by the OAM system. Victims in a set individually decide whether to transmit the RS-1.The judgement on whether or not the remote interference (RI) has stopped should be per-cell and there is no need to coordinate in this case either.

- Aggressor gNB sets:  Each aggressor cell individually concludes whether or not the applied RIM scheme has led to the cessation of RI, based on the RS-1 it receives.

For the normative phase, intra-set gNB-to-gNB coordination inside victim and aggressor gNB sets is not recommended by RAN3.

------------------------------------------End of Changes-------------------------------------------