3GPP TSG RAN WG3 Meeting #103	R3-190357

Athens, Greece, February 25th – March 1st 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0029 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Backhaul Channel Setup and Modification Procedure for IAB Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducing the backhaul channel setup and modification procedure for IAB networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IAB backhaul channel setup and modification procedure. The corresponding discussion paper is presented in R3-190356. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB backhaul channel setup and modification procedure for IAB networks not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

# 8.9.x		Backhaul RLC channel establishment procedure

The backhaul bearer corresponds to the RLC channel used for backhauling (between IAB-node and Donor-DU, or between different IAB-nodes). Backhaul RLC channels are set up and configured on the intermediate IAB-nodes along the UE’s data forwarding path by the Donor-CU at any time or when the UE’s DRB and PDU session is configured. Backhaul RLC channels can be modified when the data forwarding path change due to topology adaptation or UE handover. Moreover, the backhaul RLC channel can be modified/released on intermediate IAB-nodes when the UE bearer or the path changes.

The Donor-CU will initiate the backhaul RLC channel setup/modification procedure triggered by UE-bearer setup or the topology adaptation. Figure x shows how backhaul RLC channels are set up or modified:

1.	The donor CU determines the backhaul RLC channels that need to be established or modified on the intermediate IAB-node to support new QoS flows along the data path.

2.	The donor CU configures the UE context modification (if needed) at the Donor DU using F1 procedures and configures corresponding RLC channels at the IAB-node 2 using RRC procedures.

3.	The donor CU configures the UE context modification (if needed) at the IAB-node 2 using F1 procedures and configures corresponding RLC channels at IAB-node 1 using RRC procedures.

Figure 8.9.x-1: Signaling flow for IAB backhaul bearer establishment

-------------------------------------------End of changes ------------------------------------------