3GPP TSG-RAN WG3 Meeting #103	R3-190734

# Athens, Greece, 25th, February – 1st March 2019

Agenda Item:	9.3.13

Source:	Ericsson

Title: 	When NG-RAN node fails to set up a QoS flow for IMS voice

Document for:	Discussion and Approval

# 1	Discussion

It is specified in TS 38.413 in the PDU Session Resource Modification procedure that:

Upon reception of the PDU SESSION RESOURCE MODIFY REQUEST message to setup a QoS flow for IMS voice, if the NG-RAN node is not able to support IMS voice, the NG-RAN node shall initiate EPS fallback or RAT fallback for IMS voice procedure as specified in TS 23.501 [9] and report unsuccessful establishment of the QoS flow in the PDU Session Resource Modify Response Transfer IE with cause value "IMS voice EPS fallback or RAT fallback triggered".

It is assumed that NG-RAN node will respond the failure of the setting up of the QOS flow for IMS voice in the PDU Session Resource Modify Response Transfer IE. This is only correct under the below circumstances:

- PDU session to be modified has at least 2 QoS flows to be modified, one is the QoS flow for IMS voice;

- The setting up of the QoS flow for IMS voice fails, but the setting up the other QoS flow is successful.

In the case that the failure of setting up the QoS flow for IMS voice is within the failure of the PDU session, it is unspecified how NG-RAN node should handle.

Further, there is only one cause value in the PDU session PDU Session Resource Modify Unsuccessful Transfer IE. In our opinion, it would be important to ensure that NG-RAN node to respond with the cause value “IMS voice EPS fallback or RAT fallback triggered".

Proposal 1:   RAN3 to discuss and agree to specify how to handle the setting up of the QoS flow for IMS voice within the PDU session failure

It is noticed that there is no specification for the failure of setting up the QoS flow for IMS voice in the PDU Session Resource Setup procedure.

Proposal 2:   RAN3 to discuss and agree to include the failure handling of the setting of of the QoS flow for IMS voice in PDU Session Resource Setup procedure.

# 2	Proposal

Proposal 1:   RAN3 to discuss and agree to specify how to handle the setting up of the QoS flow for IMS voice within the PDU session failure

Proposal 2:   RAN3 to discuss and agree to include the failure handling of the setting of of the QoS flow for IMS voice in PDU Session Resource Setup procedure.

CR is submitted in [1].

# 3	Reference

[1]	R3-190735	When NG-RAN node fails to set up a QoS flow for IMS voice, Ericsson