3GPP TSG-RAN WG3 Meeting # 103	R3-191083

# Athens, Greece,  25th February - 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1305 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of TNL Address discovery for EN-DC (using legacy container) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE, Ericsson, Qualcomm Incorporated, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of this specification does not support X2 TNL address discovery for EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is introduced (E-UTRA-NR Configuration Transfer) which allows an eNB to obtain a set of X2 IP addresses for a new X2 from an already connected en-gNB (in response to a request received from another eNB and relayed via the the MME). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | X2 TNL address discovery is not supported in case of EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7, 8.1, 8.7.x1, 8.7.x2, 8.7.x3, 9.1.2.x1, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR1663, TS 38.413 CR 0065 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 7	Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Dual Connectivity. This function allows the eNB to request another eNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another en-gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the eNB or en-gNB for setup the X2 interface and implicitly perform an X2 Reset.

-	eNB Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the eNB to coordinate adaptation of mobility parameter settings with a peer eNB.

-	Mobility Robustness Optimisation. This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

-	X2 Release. This function allows an eNB to be aware that the signalling connection to a peer eNB is unavailable.

-	Message Transfer. This function allows indirect transport of X2AP messages to a peer eNB.

-	Registration. This function allows registration of eNB in case indirect transport of X2AP messages is supported.

-	Removing the X2. This function allows removing the signalling connection between two eNBs or between eNB and en-gNB in a controlled manner.

-	Inter-eNB UE Context Retrieval. This function allows retrieval of a UE context in case of resumption or re-establishment of an RRC connection.

-	Secondary RAT Data Usage Report. This function allows eNB to get the uplink and downlink data volumes for the Secondary RAT on a per E-RAB basis.

-	E-UTRA - NR Spectrum Sharing. This function allows uplink and downlink spectrum sharing between a number of E - UTRA and a number of NR cells with overlapping coverage.

-	EN-DC Configuration Transfer. This function supports en-gNB X2 TNL address discovery.

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change f) MeNB initiated SgNB Release g) SgNB initiated SgNB Release h) SgNB Counter Check i) RRC transfer j) EN-DC X2 Setup k) EN-DC Configuration Update l) EN-DC Cell Activation m) SgNB Activity Notification n) EN-DC X2 Removal o) gNB Status Indication |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Retrieval | a) Retrieve UE Context b) Data Forwarding Address Indication |
| Secondary RAT Data Usage Report | Secondary RAT Data Usage Report |
| E-UTRA – NR Spectrum Sharing | E-UTRA - NR Cell Resource Coordination |
| EN-DC Configuration Transfer | EN-DC Configuration Transfer |

# 8	X2AP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST | SGNB RELEASE REQUEST ACKNOWLEDGE | SGNB RELEASE REQUEST REJECT |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |
| EN-DC X2 Setup | EN-DC X2 SETUP REQUEST | EN-DC X2 SETUP RESPONSE | EN-DC X2 SETUP FAILURE |
| EN-DC Configuration Update | EN-DC CONFIGURATION UPDATE | EN-DC CONFIGURATION UPDATE ACKNOWLEDGE | EN-DC CONFIGURATION UPDATE FAILURE |
| EN-DC Cell Activation | EN-DC CELL ACTIVATION REQUEST | EN-DC CELL ACTIVATION RESPONSE | EN-DC CELL ACTIVATION FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| EN-DC X2 Removal | EN-DC X2 REMOVAL REQUEST | EN-DC X2 REMOVAL RESPONSE | EN-DC X2 REMOVAL FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SgNB Activity Notification | SGNB ACTIVITY NOTIFICATION |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| gNB Status Indication | GNB STATUS INDICATION |
| EN-DC Configuration Transfer | EN-DC CONFIGURATION TRANSFER |

### 8.7.x	EN-DC Configuration Transfer

#### 8.7.x.1	General

The purpose of the EN-DC Configuration Transfer procedure is to transfer the SON Configuration container, either from the eNB to the en-gNB or from the en-gNB to the eNB, in the context of en-gNB X2 TNL address discovery as described in TS 36.300 [15].

The procedure uses non UE-associated signalling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: eNB initiated EN-DC Configuration Transfer, successful operation

Figure 8.7.x.2-2: en-gNB initiated EN-DC Configuration Transfer, successful operation

The eNB initiates the procedure by sending the EN-DC CONFIGURATION TRANSFER message to an en-gNB.

The en-gNB initiates the procedure by sending the EN-DC CONFIGURATION TRANSFER message to an eNB.

#### 8.7.x.3	Abnormal Conditions

Void.

#### 9.1.2.x1	EN-DC CONFIGURATION TRANSFER

This message is sent by an eNB in order to transfer the SON Configuration container to an en-gNB, or it is sent by an en-gNB in order to transfer the SON Configuration container to an eNB.

Direction: eNB  en-gNB or en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| SON Configuration Transfer | O |  | OCTET STRING | Contains the SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |

### 9.3.3	Elementary Procedure Definitions

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

X2AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM X2AP-CommonDataTypes

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ENBConfigurationUpdate,

ENBConfigurationUpdateAcknowledge,

ENBConfigurationUpdateFailure,

ErrorIndication,

HandoverCancel,

HandoverReport,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

LoadInformation,

PrivateMessage,

ResetRequest,

ResetResponse,

ResourceStatusFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusUpdate,

RLFIndication,

SNStatusTransfer,

UEContextRelease,

X2SetupFailure,

X2SetupRequest,

X2SetupResponse,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

X2Release,

X2APMessageTransfer,

SeNBAdditionRequest,

SeNBAdditionRequestAcknowledge,

SeNBAdditionRequestReject,

SeNBReconfigurationComplete,

SeNBModificationRequest,

SeNBModificationRequestAcknowledge,

SeNBModificationRequestReject,

SeNBModificationRequired,

SeNBModificationConfirm,

SeNBModificationRefuse,

SeNBReleaseRequest,

SeNBReleaseRequired,

SeNBReleaseConfirm,

SeNBCounterCheckRequest,

X2RemovalFailure,

X2RemovalRequest,

X2RemovalResponse,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

SgNBAdditionRequest,

SgNBAdditionRequestAcknowledge,

SgNBAdditionRequestReject,

SgNBReconfigurationComplete,

SgNBModificationRequest,

SgNBModificationRequestAcknowledge,

SgNBModificationRequestReject,

SgNBModificationRequired,

SgNBModificationConfirm,

SgNBModificationRefuse,

SgNBReleaseRequest,

SgNBReleaseRequestAcknowledge,

SgNBReleaseRequestReject,

SgNBReleaseRequired,

SgNBReleaseConfirm,

SgNBCounterCheckRequest,

SgNBChangeRequired,

SgNBChangeConfirm,

SgNBChangeRefuse,

RRCTransfer,

ENDCX2SetupRequest,

ENDCX2SetupResponse,

ENDCX2SetupFailure,

ENDCConfigurationUpdate,

ENDCConfigurationUpdateAcknowledge,

ENDCConfigurationUpdateFailure,

SecondaryRATDataUsageReport,

ENDCCellActivationRequest,

ENDCCellActivationResponse,

ENDCCellActivationFailure,

ENDCPartialResetRequired,

ENDCPartialResetConfirm,

EUTRANRCellResourceCoordinationRequest,

EUTRANRCellResourceCoordinationResponse,

SgNBActivityNotification,

ENDCX2RemovalRequest,

ENDCX2RemovalResponse,

ENDCX2RemovalFailure,

DataForwardingAddressIndication,

GNBStatusIndication,

ENDCConfigurationTransfer

FROM X2AP-PDU-Contents

id-cellActivation,

id-eNBConfigurationUpdate,

id-errorIndication,

id-handoverCancel,

id-handoverReport,

id-handoverPreparation,

id-loadIndication,

id-privateMessage,

id-reset,

id-resourceStatusReporting,

id-resourceStatusReportingInitiation,

id-rLFIndication,

id-snStatusTransfer,

id-uEContextRelease,

id-x2Setup,

id-mobilitySettingsChange,

id-x2Release,

id-x2APMessageTransfer,

id-seNBAdditionPreparation,

id-seNBReconfigurationCompletion,

id-meNBinitiatedSeNBModificationPreparation,

id-seNBinitiatedSeNBModification,

id-meNBinitiatedSeNBRelease,

id-seNBinitiatedSeNBRelease,

id-seNBCounterCheck,

id-x2Removal,

id-retrieveUEContext,

id-sgNBAdditionPreparation,

id-sgNBReconfigurationCompletion,

id-meNBinitiatedSgNBModificationPreparation,

id-sgNBinitiatedSgNBModification,

id-meNBinitiatedSgNBRelease,

id-sgNBinitiatedSgNBRelease,

id-sgNBChange,

id-sgNBCounterCheck,

id-rRCTransfer,

id-endcX2Setup,

id-endcConfigurationUpdate,

id-secondaryRATDataUsageReport,

id-endcCellActivation,

id-endcPartialReset,

id-eUTRANRCellResourceCoordination,

id-SgNBActivityNotification,

id-endcX2Removal,

id-dataForwardingAddressIndication

id-gNBStatusIndication,

id-endcConfigurationTransfer

FROM X2AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode 	UNIQUE,

&criticality			Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

X2AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	X2AP-ELEMENTARY-PROCEDURE.&procedureCode		({X2AP-ELEMENTARY-PROCEDURES}),

criticality		X2AP-ELEMENTARY-PROCEDURE.&criticality			({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			X2AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({X2AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURES X2AP-ELEMENTARY-PROCEDURE ::= {

X2AP-ELEMENTARY-PROCEDURES-CLASS-1			|

X2AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-1 X2AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

reset													|

x2Setup													|

resourceStatusReportingInitiation						|

eNBConfigurationUpdate									|

mobilitySettingsChange									|

cellActivation											|

seNBAdditionPreparation									|

meNBinitiatedSeNBModificationPreparation				|

seNBinitiatedSeNBModification							|

seNBinitiatedSeNBRelease								|

x2Removal												|

retrieveUEContext										|

sgNBAdditionPreparation									|

meNBinitiatedSgNBModificationPreparation					|

sgNBinitiatedSgNBModification							|

meNBinitiatedSgNBRelease									|

sgNBinitiatedSgNBRelease									|

sgNBChange												|

endcX2Setup												|

endcConfigurationUpdate									|

endcCellActivation										|

endcPartialReset											|

eUTRANRCellResourceCoordination							|

endcX2Removal											,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-2 X2AP-ELEMENTARY-PROCEDURE ::= {

snStatusTransfer						|

uEContextRelease						|

handoverCancel							|

errorIndication							|

resourceStatusReporting					|

loadIndication							|

privateMessage							|

rLFIndication							|

handoverReport							|

x2Release								|

x2APMessageTransfer						|

seNBReconfigurationCompletion			|

meNBinitiatedSeNBRelease				|

seNBCounterCheck						|

sgNBReconfigurationCompletion			|

sgNBCounterCheck							|

rRCTransfer								|

secondaryRATDataUsageReport				|

sgNBActivityNotification					|

dataForwardingAddressIndication			|

gNBStatusIndication						|

endcConfigurationTransfer				,

...

}

>>>>>   skip unchanged text in same section   <<<<<<

dataForwardingAddressIndication	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataForwardingAddressIndication

PROCEDURE CODE			id-dataForwardingAddressIndication

CRITICALITY				ignore

}

gNBStatusIndication		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBStatusIndication

PROCEDURE CODE			id-gNBStatusIndication

CRITICALITY				ignore

}

endcConfigurationTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCConfigurationTransfer

PROCEDURE CODE			id-endcConfigurationTransfer

CRITICALITY				ignore

}

END

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

AerialUEsubscriptionInformation,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RLC-Status,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UEsToBeResetList,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

SplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

SplitSRB,

UENRMeasurement,

EN-DC-ResourceConfiguration,

TAC,

NRFreqInfo,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

RLCMode,

GBR-QosInformation,

DRB-ID,

FiveGS-TAC,

SULInformation,

Packet-LossRate,

ResourceType,

DataTrafficResourceIndication,

SpectrumSharingGroupID,

RRC-Config-Ind,

SGNB-Addition-Trigger-Ind,

UserPlaneTrafficActivityReport,

ERABActivityNotifyItemList,

PDCPSnLength,

Subscription-Based-UE-DifferentiationInfo,

LCID,

DuplicationActivation,

GNBOverloadInformation,

NewDRBIDrequest,

SONConfigurationTransfer

FROM X2AP-IEs

>>>>>   skip unchanged text in same section   <<<<<<

id-dLPDCPSnLength,

id-secondarysgNBDLGTPTEIDatPDCP,

id-secondarymeNBULGTPTEIDatPDCP,

id-lCID,

id-duplicationActivation,

id-GNBOverloadInformation,

id-new-drb-ID-req,

id-SONConfigurationTransfer,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxofNRNeighbours,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands

FROM X2AP-Constants;

>>>>>   skip unchanged text in same section   <<<<<<

-- **************************************************************

--

#### -- GNB STATUS INDICATION

--

-- **************************************************************

GNBStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ { GNBStatusIndicationIEs} },

...

}

GNBStatusIndicationIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GNBOverloadInformation		CRITICALITY ignore	TYPE GNBOverloadInformation		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- EN-DC CONFIGURATION TRANSFER

--

-- **************************************************************

ENDCConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationTransfer-IEs}},

...

}

ENDCConfigurationTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransfer	CRITICALITY ignore	TYPE SONConfigurationTransfer			PRESENCE optional},

...

}

END

### 9.3.5	Information Element definitions

>>>>>   skip unchanged text in same section   <<<<<<

ShortMAC-I ::= BIT STRING (SIZE(16))

SGNB-Addition-Trigger-Ind	 ::= ENUMERATED {

sn-change,

inter-eNB-HO,

intra-eNB-HO,

...

}

SONConfigurationTransfer		OCTET STRING

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		SpecialSubframePatterns,

cyclicPrefixDL				CyclicPrefixDL,

cyclicPrefixUL				CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { {SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

SpecialSubframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-handoverCancel												ProcedureCode ::= 1

id-loadIndication												ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-snStatusTransfer												ProcedureCode ::= 4

id-uEContextRelease												ProcedureCode ::= 5

id-x2Setup														ProcedureCode ::= 6

id-reset														ProcedureCode ::= 7

id-eNBConfigurationUpdate										ProcedureCode ::= 8

id-resourceStatusReportingInitiation							ProcedureCode ::= 9

id-resourceStatusReporting										ProcedureCode ::= 10

id-privateMessage												ProcedureCode ::= 11

id-mobilitySettingsChange										ProcedureCode ::= 12

id-rLFIndication												ProcedureCode ::= 13

id-handoverReport												ProcedureCode ::= 14

id-cellActivation												ProcedureCode ::= 15

id-x2Release													ProcedureCode ::= 16

id-x2APMessageTransfer											ProcedureCode ::= 17

id-x2Removal													ProcedureCode ::= 18

id-seNBAdditionPreparation										ProcedureCode ::= 19

id-seNBReconfigurationCompletion								ProcedureCode ::= 20

id-meNBinitiatedSeNBModificationPreparation						ProcedureCode ::= 21

id-seNBinitiatedSeNBModification								ProcedureCode ::= 22

id-meNBinitiatedSeNBRelease										ProcedureCode ::= 23

id-seNBinitiatedSeNBRelease										ProcedureCode ::= 24

id-seNBCounterCheck												ProcedureCode ::= 25

id-retrieveUEContext											ProcedureCode ::= 26

id-sgNBAdditionPreparation										ProcedureCode ::= 27

id-sgNBReconfigurationCompletion								ProcedureCode ::= 28

id-meNBinitiatedSgNBModificationPreparation						ProcedureCode ::= 29

id-sgNBinitiatedSgNBModification								ProcedureCode ::= 30

id-meNBinitiatedSgNBRelease										ProcedureCode ::= 31

id-sgNBinitiatedSgNBRelease										ProcedureCode ::= 32

id-sgNBCounterCheck												ProcedureCode ::= 33

id-sgNBChange													ProcedureCode ::= 34

id-rRCTransfer													ProcedureCode ::= 35

id-endcX2Setup													ProcedureCode ::= 36

id-endcConfigurationUpdate										ProcedureCode ::= 37

id-secondaryRATDataUsageReport									ProcedureCode ::= 38

id-endcCellActivation											ProcedureCode ::= 39

id-endcPartialReset												ProcedureCode ::= 40

id-eUTRANRCellResourceCoordination								ProcedureCode ::= 41

id-SgNBActivityNotification										ProcedureCode ::= 42

id-endcX2Removal												ProcedureCode ::= 43

id-dataForwardingAddressIndication								ProcedureCode ::= 44

id-gNBStatusIndication											ProcedureCode ::= 45

id-endcConfigurationTransfer									ProcedureCode ::= AA

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 65535

maxEARFCNPlusOne							INTEGER ::= 65536

newmaxEARFCN								INTEGER ::= 262143

maxInterfaces								INTEGER ::= 16

maxCellineNB								INTEGER ::= 256

maxnoofBands								INTEGER ::= 16

maxnoofBearers								INTEGER ::= 256

maxNrOfErrors								INTEGER ::= 256

maxnoofPDCP-SN								INTEGER ::= 16

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofForbLACs								INTEGER ::= 4096

maxnoofForbTACs								INTEGER ::= 4096

maxnoofBPLMNs								INTEGER ::= 6

maxnoofNeighbours							INTEGER ::= 512

maxnoofPRBs									INTEGER ::= 110

maxPools									INTEGER ::= 16

maxnoofCells								INTEGER ::= 16

maxnoofMBSFN								INTEGER ::= 8

maxFailedMeasObjects						INTEGER ::= 32

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofTAforMDT								INTEGER ::= 8

maxnoofMBMSServiceAreaIdentities			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofCoMPHypothesisSet					INTEGER ::= 256

maxnoofCoMPCells							INTEGER ::= 32

maxUEReport									INTEGER ::= 128

maxCellReport								INTEGER ::= 9

maxnoofPA									INTEGER ::= 3

maxCSIProcess								INTEGER ::= 4

maxCSIReport								INTEGER ::= 2

maxSubband									INTEGER ::= 14

maxofNRNeighbours							INTEGER ::= 1024

maxCellinengNB								INTEGER ::= 16384

--	maxnoofNRCarriers						INTEGER ::= 32

maxnooftimeperiods							INTEGER ::= 2

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofTAforQMC								INTEGER ::= 8

maxnoofPLMNforQMC							INTEGER ::= 16

maxUEsinengNBDU								INTEGER ::= 8192

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoNRcellsSpectrumSharingWithE-UTRA		INTEGER ::= 64

maxnoofNrCellBands							INTEGER ::= 32

maxnoofBluetoothName						INTEGER ::= 4

maxnoofWLANName								INTEGER ::= 4

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

id-CellInformation															ProtocolIE-ID ::= 6

id-CellInformation-Item														ProtocolIE-ID ::= 7

id-New-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 9

id-Old-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 10

id-TargetCell-ID															ProtocolIE-ID ::= 11

id-TargeteNBtoSource-eNBTransparentContainer								ProtocolIE-ID ::= 12

>>>>>   skip unchanged text in same section   <<<<<<

id-NRCGI																	ProtocolIE-ID ::= 322

id-MeNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 323

id-SgNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 324

id-new-drb-ID-req															ProtocolIE-ID ::= 325

id-SONConfigurationTransfer													ProtocolIE-ID ::= XXX

END