3GPP TSG-RAN3 Meeting #103-Bis	R3-191303

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0081 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TDD configuration exchange over Xn for CLI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For network coordination mechanism(s) for CLI, RAN1 #96 made the following agreements: - Agreements on intended UL/DL configuration - For “Intended DL/UL configuration”, per-slot based OFDM symbol level DL/UL region indication is supported. - Periodicity and numerology (i.e. subcarrier spacing, CP length) are configured. - A list containing multiple entries of the following: - Information for indicating slot within a periodicity is configured. - DL/UL region indication (i.e. all DL or all UL or number of DL symbols and/or number of UL symbols in a slot) is configured for each slot The TDD-UL-DL-ConfigCommon IE as defined in TS 38.331 includes this slot format information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposes to add following IEs into Served Cell Information NR IE: - Intended TDD configuration (TDD-UL-DL-ConfigCommon) - Cyclic Prefix |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CLI resource coordination is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.11, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| NR CGI | M |  | 9.2.2.7 |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| CHOICE NR-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>Intended TDD Configuration | O |  | OCTET STRING | Contains TDD-UL-DL-ConfigCommon as defined in TS 38.331 [10] |
| >>>Cyclic Prefix | O |  | ENUMERATED (Extended, ...) | Normal cyclic prefix is assumed if this IE is not included |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. |
| Connectivity Support | M |  | 9.2.2.28 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

========================================Next Change ================================

NRModeInfoTDD ::= SEQUENCE {

nrFrequencyInfo			NRFrequencyInfo,

nrTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension			ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

intended-TDD-Config		OCTET STRING				OPTIONAL,

cyclicPrefix			ENUMERATED {Extended, ...}	OPTIONAL,

...

}