3GPP TSG RAN WG3 Meeting #103bis	R3-191360

Xi’an, P.R. China, April 8th – 12th 2019

Agenda Item:	13.2.1.2

Source:	Ericsson

Title:	Reuse of UE Context Management Procedures for IAB-node MT Functionality Contexts

Document for:	Agreement

# Introduction

It has been agreed that IAB node will initially connect using RRC signalling like a UE. Furthermore, at the RAN3#103 meeting it was agreed that the CN function will indicate to the CU-CP in the NG UE context setup when the IAB (MT) has been authorized as an IAB node. Once the IAB node has been authorized, the CU-CP could initiate the setup of BH RLC channels to the IAB node, by communicating over F1-AP to the IAB node parent.

A question that arises is how this signalling should be done. Two major options are considered:

- Specify new IAB MT Functionality Context Management Procedures in F1-AP, or

- Reuse the existing UE Context Management Procedures in F1-AP, and only add relevant IAB-specific IEs.

This paper analyses the two options and proposes to reuse the existing UE Context Management Procedures.

# The content of F1 UE context

The table below shows the content of the F1 UE Context Setup Request message. For each field, we comment on the applicability to the IAB MT functionality context.

| IE/Group Name | Presence | Range | Applicability to IAB (MT) |
| --- | --- | --- | --- |
| Message Type | M |  | Yes |
| gNB-CU UE F1AP ID | M |  | Yes |
| gNB-DU UE F1AP ID | O |  | Yes |
| SpCell ID | M |  | Yes |
| ServCellIndex | M |  | Yes |
| SpCell UL Configured | O |  | May depend on DC support IAB nodes |
| Candidate SpCell List |  | 0..1 | Yes |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> | Yes |
| >>Candidate SpCell ID | M |  | Yes |
| CU to DU RRC Information | M |  | Most likely needed |
| DRX Cycle | O |  | Connected mode DRX may be used if desired. |
| Resource Coordination Transfer Container | O |  | May depend on DC support IAB nodes |
| SCell To Be Setup List |  | 0..1 | Yes, CA should be supported for IAB (MT) |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> | Yes |
| >>SCell ID | M |  | Yes |
| >>SCellIndex | M |  | Yes |
| >>SCell UL Configured | O |  | Yes |
| >>servingCellMO | O |  | Yes |
| SRB to Be Setup List |  | 0..1 | Yes, SRBs should be supported for IAB (MT) nodes |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> | Yes |
| >>SRB ID | M |  | Yes |
| >>Duplication Indication | O |  | May depend on DC support IAB nodes |
| DRB to Be Setup List |  | 0..1 | Yes. DRBs could optionally be supported for IAB (MTs) |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> | Yes |
| >>DRB ID | M |  | Yes |
| >>CHOICE QoS Information | M |  | Yes |
| >>>E-UTRAN QoS | M |  | Yes |
| >>>DRB Information |  | 1 | Yes |
| >>>>DRB QoS | M |  | Yes |
| >>>>S-NSSAI | M |  | Yes |
| >>>>Notification Control | O |  | Yes |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> | Yes |
| >>>>>QoS Flow Identifier | M |  | Yes |
| >>>>>QoS Flow Level QoS Parameters | M |  | Yes |
| >>>>>QoS Flow Mapping Indication | O |  | Yes |
| >>UL UP TNL Information to be setup List |  | 1 | Yes |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> | Yes |
| >>>>UL UP TNL Information | M |  | Yes |
| >> RLC Mode | M |  | Yes |
| >> UL Configuration | O |  | May depend on how DC is supported for IAB nodes |
| >>Duplication Activation | O |  | May depend on how DC is supported for IAB nodes |
| >> DC Based Duplication Configured | O |  | May depend on how DC is supported for IAB nodes |
| >>DC Based Duplication Activation | O |  | May depend on how DC is supported for IAB nodes |
| >>DL PDCP SN length | M |  | May depend on how DC is supported for IAB nodes |
| >>UL PDCP SN length | O |  | May depend on how DC is supported for IAB nodes |
| Inactivity Monitoring Request | O |  | Could be supported for IAB (MT) nodes |
| RAT-Frequency Priority Information | O |  | Could be supported for IAB (MT) nodes |
| RRC-Container | O |  | Yes |
| Masked IMEISV | O |  | Could be supported for IAB (MT) nodes |
| Serving PLMN | O |  | Yes |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Could be supported for IAB (MT) nodes |
| RRC Delivery Status Request | O |  | Could be supported for IAB (MT) nodes |
| Resource Coordination Transfer Information | O |  | May depend on how DC is supported for IAB nodes |
| servingCellMO | O |  | Yes |

As can be seen from the above text, most of the parameters in the UE context are also applicable to the IAB MT functionality context. Therefore, it is beneficial to reuse the UE context messages and procedures also for managing the IAB MT functionality contexts. Any IAB-specific information such as RLC BH channel configuration could easily be added as optional IAB specific IEs. Another benefit of re-using the UE Context Management procedures for IAB MT functionality context management is that this will automatically mean that also other procedures, such as RRC transfer, will also work.

Most of the parameters currently in the F1 UE Context Setup are also applicable for IAB MT functionality context.

Re-using F1 UE Context Management for IAB MT functionality context will minimize standardization effort and ensure that also other procedures, such as RRC transfer, will work.

The F1 UE Context Management procedures should be reused for IAB MT functionality context management. Any IAB MT functionality-specific aspects, such as RLC BH channels, could be signalled as a separate optional IEs in the UE Context Management messages.

Agree to CRs to TS 38.470 [1] and TS 38.473 [2] capturing this.

# Conclusions

In earlier sections we made the following observations:

Observation 1	Most of the parameters currently in the F1 UE Context Setup are also applicable for IAB MT functionality context.

Observation 2	Re-using F1 UE Context Management for IAB MT functionality context will minimize standardization effort and ensure that also other procedures, such as RRC transfer, will work.

Based on the discussion in earlier sections we propose the following:

Proposal 1	The F1 UE Context Management procedures should be reused for IAB MT functionality context management. Any IAB MT functionality-specific aspects, such as RLC BH channels, could be signalled as a separate optional IEs in the UE Context Management messages.

- Proposal 2	Agree to CRs to TS 38.470 [1] and TS 38.473 [2] capturing this.

# References

[1] R3-191354; “CR TS 38.470 IAB General Aspects”, Ericsson, RAN3#103bis, April 2019.

[2] R3-191361; “CR TS 38.473 Reuse of UE Context Management Procedures for IAB-node MT functionality Contexts”, Ericsson, RAN3#103bis, April 2019.