3GPP TSG-RAN WG3 #103bis                                              	　　　　　　　　    　　　　　　R3-191411

Xi’an, China, 8th – 12th April 2019

Source: 	KDDI Corporation

Title:	Consideration on IAB setup procedure

Tdoc Type:	Discussion

Agenda Item:	13.2.1.1

Document for:	Discussion and decision

Release:	Rel-16

Introduction

In RAN3#103bis, RAN3 identified the baseline for “Time line for IAB setup” [1]. In this contribution we try to identify the detailed procedures for “Time line for IAB setup”. We also provide text proposal for related signaling flows for IAB-node DU setup and IAB Route Modification.

First, we show example signalling flows for the cases below.

- IAB-node DU setup (connected directly to the donor-DU)

- IAB Route Modification (additional BH RLC channel establishment triggered by UE connections)

- IAB-node DU setup (connected to the donor-DU via another IAB-node)

Next, we make some proposals

- Proposals related to signalling flows for IAB-node DU setup and IAB Route Modification

Discussion

  - Example signaling flows

In this section, we example signalling flows for or IAB-node DU setup and IAB Route Modification.

IAB-node DU setup (connected directly to the donor-DU)

Figure1. IAB-node DU setup (connected directly to the donor-DU) procedure

- Backhaul RLC channel is established by the following procedure.

  - The Donor-CU sends an DL RRC MESSAGE TRANSFER to the Donor-DU.

  - The Donor-DU sends RRCReconfiguration to IAB node1 to create a new RLC entity which works as Backhaul RLC channel between IAB node1 and Donor-DU.

  - The IAB node1 sends RRCReconfigurationComplete to Donor-DU.

  - The Donor-DU sends DL RRC MESSAGE TRANSFER to the Donor-CU.

- IP address of IAB node1 is assigned by the following procedure,

  - IAB node1 sends DHCP discovery to the Donor-DU, the Donor-DU forwards it to Donor-CU.

  - The donor-CU sends DHCP offer to the Donor-DU, the Donor-DU forwards it to IAB node1

  - IAB node1 sends DHCP request to the Donor-DU, the Donor-DU forwards it to Donor-CU.

  - The donor-CU sends DHCP ack to the Donor-DU, the Donor-DU forwards it to IAB node1

- Routing table for southbound in the donor-DU is updated by the following procedure

  - The donor-CU sends ROUTING INFORMATION UPDATE REQ containing the table1 to the Donor-DU

  - The donor-DU updates its routing table for southbound and sends ROUTING INFORMATION UPDATE CNF to the donor-CU

- Routing table for northbound in IAB node1 is updated by the following procedure

  - The donor-CU sends ROUTING INFORMATION UPDATE REQ containing the table1 to the Donor-DU, the Donor-DU forwards it to IAB node1

  - IAB node1 updates its routing table for northbound and sends ROUTING INFORMATION UPDATE CNF to the Donor-DU, the Donor-DU forwards it to the donor-CU

IAB Route Modification (additional BH RLC channel establishment triggered by UE connections)

Figure2. BH RLC channel establishment triggered by triggered by UE connection

- Backhaul RLC channel is established. The established channel is used to forward user data for UE1 and UE2

- Routing table for southbound in the donor-DU is updated. Updated information is marked as red in the table2.

- Routing table for northbound in IAB node1 is updated. Updated information is marked as red in the table2.

IAB-node DU setup (connected to the donor-DU via another IAB-node)

- Backhaul RLC channel is established. The established channel is used to forward IAB node2’s F1 messages

- Routing table for southbound in the donor-DU is updated. Updated information is marked as red in the table3.

- Routing table for northbound in IAB node1 is updated. Updated information is marked as red in the table3.

- Routing table for southbound in IAB node1 is updated. Updated information is marked as red in the table4.

- Routing table for southbound in IAB node1 is updated. Updated information is marked as red in the table4.

  - Observations and proposals

Observation form the above signaling flows, we propose Proposal1 and Propsal2 below.

Proposal1: 	IAB-node DU setup procedure is taken apart to the following five sub procedures

- Backhaul RLC channel (new RLC entity) for F1 communication establishment

- IP address assignment

- Routing information update in newly added IAB node

- Routing information update in the donor-DU

- Routing information update in the affected intermediate IAB nodes

Proposal2: 	IAB Route Modification procedure is taken apart to the following five sub procedures

- Backhaul RLC channel (new RLC entity) for user data forwarding

- Routing information update in newly added IAB node

- Routing information update in the donor-DU

- Routing information update in the affected intermediate IAB nodes

TP for signaling flow (TS38.401)

Proposal7: 	RAN2/3 agree to have the following TP for IAB-node DU setup and IAB Route Modification.

>>>> Skip

- 8.x.1.3		IAB-node DU setup

The signalling flow for IAB-node DU setup is shown in Figure 8.x.1.3.

Figure 8.x.1.3: IAB-node integration procedure (SA-node with NGC)

- Backhaul RLC channel is established by the following procedure.

  - The Donor-CU sends an DL RRC MESSAGE TRANSFER to the Donor-DU.

  - Intermediate IAB-nodes relay the message to parent node, IAB node (x-1), and IAB node (x-1) sends RRCReconfiguration to IAB node (x) to create a new RLC entity which works as Backhaul RLC channel between IAB node (x) and IAB node (x-1).

  - IAB node (x) sends RRCReconfigurationComplete to IAB node (x-1).

  - IAB node (x-1) sends DL RRC MESSAGE TRANSFER to the parent IAB-node. Intermediate IAB-nodes relay it to the Donor-CU.

- IP address of IAB node1 is assigned by the following procedure,

  - IAB node (x) sends DHCP discovery to IAB node (x-1), intermediate IAB nodes relay it to Donor-CU.

  - The Donor-CU sends DHCP offer to the Donor-DU, intermediate IAB nodes relay it to IAB node (x)

  - IAB node (x) sends DHCP request to IAB node (x-1), intermediate IAB nodes relay it to Donor-CU.

  - The Donor-CU sends DHCP ack to the Donor-DU, intermediate IAB nodes relay it to IAB node (x).

- Routing tables for IAB node (x), IAB node (x-1), intermediate IAB nodes are updated by the following procedure

  - The donor-CU sends ROUTING INFORMATION UPDATE REQ to IAB node (x), IAB node (x-1) and intermediate IAB nodes. The ROUTING INFORMATION UPDATE REQ messages contain the routing table which includes affected paths by attaching IAB node (x).

  - Each receiving IAB node and the Donor-DU sends ROUTING INFORMATION UPDATE CNF to the Donor-CU.

  - Intermediate IAB nodes can relay ROUTING INFORMATION UPDATE REQ/CNF based on the previous routing information.

>>>> Skip

- 8.x.2.1 	IAB Route Modification

The signaling flow for IAB Route Modification is shown in Figure 8.x.2.1.

Figure 8.x.2.1: IAB Route Modification

- Backhaul RLC channel is added/released/modified by the following procedure.

  - The Donor-CU sends an DL RRC MESSAGE TRANSFER to the Donor-DU.

  - Intermediate IAB-nodes relay the message to parent node, IAB node (x-1), and IAB node (x-1) sends RRCReconfiguration to IAB node (x) to create a new RLC entity which works as Backhaul RLC channel between IAB node (x) and IAB node (x-1).

  - IAB node (x) sends RRCReconfigurationComplete to IAB node (x-1).

  - IAB node (x-1) sends DL RRC MESSAGE TRANSFER to the parent IAB-node. Intermediate IAB-nodes relay it to the Donor-CU.

- Routing tables for IAB node (x), IAB node (x-1), intermediate IAB nodes are updated by the following procedure

  - The donor-CU sends ROUTING INFORMATION UPDATE REQ to IAB node (x), IAB node (x-1) and intermediate IAB nodes. The ROUTING INFORMATION UPDATE REQ messages contain the routing table which includes affected paths by route modification.

  - Each receiving IAB node and the Donor-DU sends ROUTING INFORMATION UPDATE CNF to the Donor-CU.

  - Intermediate IAB nodes can relay ROUTING INFORMATION UPDATE REQ/CNF based on the previous routing information.

References

[1] R3-191006 offline summary repotCB#55 IAB setup		Qualcomm Incorporated