3GPP TSG-RAN WG3 Meeting #103bis	R3-191701

# Xian, P.R. China, 8th – 12th April 2019

Agenda Item:	9.2

Source:	Ericsson

Title: 	ERROR INDICATION Message in S1AP

Document for:	Discussion and Approval

# 1	Discussion

The Error Indication procedure is used for one node receiving the message to indicate to the node sending the message when there is a error related to unknown, protocol data, when there is no appropriate response message to feedback the error.

When the message is sent for a particular UE, but without any UE APID, the receiving node cannot feedback the error ping point to the right UE. For example, in RETRIEVE UE INFORMATUION message, only S-TMSI is used. If S-TMSI is unknown by MME, it would be benefit to feedback to eNB what went wrong. Thus we propose to add S-TMSI in ERROR INDICATION.

#### 9.1.4.21	RETRIEVE UE INFORMATION

The message is sent by the eNB to request UE information over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| S-TMSI | M |  | 9.2.3.6 |  | YES | reject |

#### 9.1.8.3	ERROR INDICATION

This message is sent by both the MME and the eNB and is used to indicate that some error has been detected in the node.

Direction: MME  eNB and eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | O |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | O |  | 9.2.3.4 |  | YES | ignore |
| Cause | O |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

Proposal 1:   RAN3 to agree to add S-TMSI in ERROR INDICATION

# 2	Proposal

Proposal 1:   RAN3 to agree to add S-TMSI in ERROR INDICATION