3GPP TSG-RAN WG3 meeting #103bis 	R3-191837

Xi‘an, China, 8th April– 12th April, 2019

Agenda Item:	13.2.1.2

Source: 	Huawei

Title: 		Backhaul RLC bearer management

Document for:		Discussion and Decision

- Introduction

As we know, in an IAB network, the UP and CP packet will be mapped to backhaul link RLC channels in order to provide appropriate QoS guarantees. Referring to the discussion in the study item phase, an example of the backhaul bearer establishment/modification procedure is shown in the section 9.5.2 of [1]. This example just provides a framework for the configuration signalling. Additional details for the management of backhaul link RLC channels haven’t been decided yet.

In last RAN2 #105 meeting [2], RAN 2 agreed the following assumptions about the backhaul RLC channel management.

R2 assumes that Donor CU is controlling the setup and modification of all backhaul channels in the IAB network below the Donor.

R2 assumes that a separate BH RLC channel should be setup for each UE DRB with one-to-one bearer mapping.

R2 assumes that For UE DRB with many-to-one bearer mapping, a BH RLC channel associated with IAB node existing BH RLC channel might be reused as BH RLC channel to forward traffic of UE DRB (e.g. if the BH RLC Channel supports the required UE DRB QoS).

In this contribution, we are going to analyse the procedure for the management wireless backhaul link RLC channels (or RLC bearers).

- Discussion

Analysis of the configuration aspects for BH RLC channels

There are two related aspects that needs to be considered for the management of each BH RLC channel. The first one is the setup/modification/release of the BH RLC channel, the second is the mapping rules related to the BH RLC channel.

In last RAN2 meeting, it has been agreed that “RAN2 confirms that routing and bearer mapping (e.g. mapping of BH RLC channels) are adaptation layer functions” [2]. Therefore, the mapping rules for BH RLC channels should be included in the adaptation layer configuration, assuming that the bearer mapping is controlled by the donor CU in a centralized way. The adaptation layer is part of the L2 protocol. Thus it make sense that the configuration of the adaptation layer, at least bear mapping related configurations, should be discussed and decided by RAN2. However, if the bearer mapping is configured for the DU part (e.g the downlink bearer mapping), RAN3 should decide the F1AP message about the mapping rules of BH RLC channels.

As shown in Figure 1, for each BH link, the RLC channel related configuration involves two nodes, i.e. the DU part of the parent node, and the MT part of the child node. For the DU part of the parent node, according to the agreed assumption that “R2 assumes that Donor CU is controlling the setup and modification of all backhaul channels in the IAB network below the Donor”, the CU-CP will use F1AP message to define a list of BH RLC channels to be setup/modified/released, which should include the QoS parameters related to each BH RLC channel. Some new kind of F1AP message may need to be defined since the BH RLC channel didn’t corresponding to any specific BH DRB/SRB, we may take the UE context management messages as template, but modifications are still necessary.

For the MT part of the child node, the BH channel configuration is mainly concerned with the configuration of the lower layers (RLC/MAC/PHY) to match the peer entity located at the DU part of the parent node. Thus the configuration for the MT part of an IAB node during the BH RLC channel setup/modification/release should be determined by the corresponding DU part of its parent node, just as is the case with an access link where the RLC-BearerConfig for the UE is generated by the gNB-DU. The DU part of parent node should send the RRC message content related to the BH RLC configuration for the MT part of child IAB node towards the CU-CP, and then the CU-CP can build an appropriate RRC message (e.g. RRC reconfiguration request) and forward to the MT part of child IAB node.  It is worth noting that the F1AP related configuration should be discussed and decided by RAN3, but the detailed configuration for the MT part of the BH RLC channel setup/modification/release can be discussed and decided by RAN2.

Observation 1: The configuration of BH RLC channel involves the DU part of parent node, and the MT part of the child node.

Observation 2: It is straightforward to use F1AP messages for requesting the setup of BH RLC channels and related QoS, at the DU part of parent node, as well as to use RRC messaging to configure BH RLC channels for the MT part of child IAB node.

Proposal 1: F1-AP message is used to carry the BH RLC channel configuration for the DU part of parent node. Detailed contents of the new F1-AP message are FFS.

Proposal 2: The DU of the parent node generates the configuration for the corresponding MT part of child IAB node, encapsulates the configuration IEs in the appropriate F1AP message container towards the CU-CP. The CU-CP builds the corresponding RRC message, and sends the configuration to the MT of child IAB node.

Procedure for the BH RLC channel management

Figure 1. The example for backhaul RLC channel management

As shown in Figure 1, an example about the backhaul RLC management is provided, and the procedure for the BH RLC channel management in the BH link between donor DU and the IAB node 1 is introduced as follows.

- CU-CP requests the donor DU part to configure the BH RLC channel via F1AP messaging, carries information about the RLC channels to be setup/modified/released, e.g. an appropriate ID to identify the BH RLC channel, QoS information, and possibly mapping rules related to the RLC channel, etc.

- Donor DU send response to CU-CP via F1AP message, which carries the configuration of the corresponding RLC channel (e.g. some IE similar to RLC-BearerConfig) which needs to be setup/modified/released for the MT of IAB node 1.

- CU-CP send RRC message (e.g. RRC reconfiguration request) to IAB node 1 to configure the BH RLC channel between IAB node 1 and donor DU. The structure of this configuration IE may be similar to the RLC-BearerConfig in current specification TS 38.331[3]. More details about the configuration IE still need to be analyzed and defined(e.g. there is no PDCP layer in the backhaul link and the terms DRB/SRB ID may not suitable to identify the backhaul RLC channel, etc.)

- IAB node 1 responds RRC Reconfiguration Complete towards CU-CP.

Similarly, the BH RLC channel between IAB node 1 and IAB node 2 will be setup/modified/released in another 4 step procedure (step 5 - step 8).

Proposal 3: It is left for RAN2 to decide the structure of configuration IE of BH RLC channel for the IAB-MT which works as child node.

When to trigger the BH RLC channel management

Based on the second bullet of the agreed RAN2 assumptions listed in the section 1, 1:1 mapping related BH RLC bearers should be setup/modified/released when the mapped UE bearer needs to be setup/modified/released. This means that UE bearer management is a reasonable trigger for the management of BH RLC channels.

Proposal 4: UE bearer management is the trigger for BH RLC channel management at least for 1:1 bearer mapping.

In addition, according to the third bullet, for the N:1 bearer mapping case, the BH RLC channel may multiplex many UE bearers. Thus, we can imagine that for a BH RLC channel supports N:1 mapping, the setup/modification/release of such BH RLC channel can be triggered by some specific UE bearer management event, or totally independently of any specific UE bearer.

Proposal 5: Management of BH RLC channel for N:1 mapping can be triggered by UE bearer management, or can totally independent of any specific UE bearer.

For the former case, the BH RLC bearer used for N:1 mapping may exist already. When a new UE bearer which can be mapped to this BH RLC channel is setup, the CU-CP may find that the BH RLC channel can no longer meet the aggregate QoS requirements of all the multiplexed UE bearers (including the new one). Then CU-CP may trigger a modification procedure for the BH RLC channel to change its QoS parameters.

For the latter case, the BH RLC channel for N:1 mapping can be configured before any UE bearer is setup, e.g. the BH RLC channel may be established before any UE connection to the IAB node. Furthermore, the BH RLC channel between an IAB node and its parent node can be setup before or after its DU part establishes an F1* connection towards the CU. For example, the RLC channel used for F1 setup procedure needs to be configured in IAB-MT and its parent node, before the IAB-DU setup. While the BH RLC channel used for UE’s data transmission can be setup after the IAB-DU setup since there is no UE traffic before the IAB-DU setup.

Proposal 6: At least one BH RLC channel between IAB-node MT and its parent node DU is configured prior to IAB-node DU setup, in order to carry F1 Setup procedure messages between IAB-DU and the IAB donor CU.

Conclusion

This paper mainly discusses the management of RLC channel in backhaul link. Based on the above discussion, we have the following observations and proposals:

Observation 1: The configuration of BH RLC channel involves the DU part of parent node, and the MT part of the child node.

Observation 2: It is straightforward to use F1AP messages for requesting the setup of BH RLC channels and related QoS, at the DU part of parent node, as well as to use RRC messaging to configure BH RLC channels for the MT part of child IAB node.

Proposal 1: F1-AP message is used to carry the BH RLC channel configuration for the DU part of parent node. Detailed contents of the new F1-AP message are FFS.

Proposal 2: The DU of the parent node generates the configuration for the corresponding MT part of child IAB node, encapsulates the configuration IEs in the appropriate F1AP message container towards the CU-CP. The CU-CP builds the corresponding RRC message, and sends the configuration to the MT of child IAB node.

Proposal 3: It is left for RAN2 to decide the structure of configuration IE of BH RLC channel for the IAB-MT which works as child node.

Proposal 4: UE bearer management is the trigger for BH RLC channel management at least for 1:1 bearer mapping.

Proposal 5: Management of BH RLC channel for N:1 mapping can be triggered by UE bearer management, or can totally independent of any specific UE bearer.

Proposal 6: At least one BH RLC channel between IAB-node MT and its parent node DU is configured prior to IAB-node DU setup, in order to carry F1 Setup procedure messages between IAB-DU and the IAB donor CU.

- Reference

- 3GPP TR 38.874 v16.0.0, Study on Integrated Access and Backhaul.

- Chairman notes of 3GPP RAN WG2 #105 meeting.

- 3GPP TS38.331 v15.4.0, Radio Resource Control (RRC) protocol specification.