3GPP TSG-RAN WG3 meeting #103bis 	R3-191838

Xi‘an, China, 8th April– 12th April, 2019

Agenda Item:	13.2.1.2

Source: 	Huawei

Title: 		Backhaul RLC bearer management

Document for:	other

- Text proposal for TS 38.401

- 8.9.x Backhaul RLC channel management over F1

The backhaul RLC channel management (including setup/modification/release of RLC channel) is controlled by the IAB-donor-CU. The BH RLC channel management (setup/modification/release) can be triggered by UE radio bearer management (setup/modification/release), or irrelevant to any specific UE bearer (the details of the trigger event are FFS). The procedure for the BH RLC channel management in the BH link between IAB-donor-DU and the IAB-node 1 is shown in Figure 8.9.x-1 in step 1-step 4.

1.	CU-CP requests the donor DU part to configure the BH RLC channel via F1AP messaging, carries information about the RLC channels to be setup/modified/released, e.g. an appropriate ID to identify the BH RLC channel, QoS information, and possibly mapping rules related to the RLC channel, etc.

2.	The IAB-donor-DU sends response to CU-CP via F1AP message. Which carries the configuration of the corresponding RLC channel (e.g. some IE similar to RLC-BearerConfig) which needs to be setup/modified/released for the MT of IAB node 1.

3.	CU-CP sends RRCReconfiguration to the MT part of IAB-node 1 to configure the BH RLC channel between the IAB-node 1 and the IAB-donor-DU.

4.	The IAB-node 1 responds RRCReconfigurationComplete towards CU-CP.

Similarly, the BH RLC channel between IAB node 1 and IAB node 2 will be setup/modified/released in another 4 steps procedure (step 5-step 8).

Figure 8.9.x-1. Backhaul RLC channel management