3GPP TSG RAN WG3 Meeting #103bis	R3-192063

Xi’an, P.R. China, April 8th – 12th 2019

Agenda Item:	13.2.1

Source:	Ericsson

Title:	Summary of offline discussion on IAB Node Release Procedure

Document for:	Agreement

# Problem statement

CB: # 35_IABrelease

- clarify details in case of multi-hop if possible (intermediate nodes)

- check details

- attempt st2 TP

The issue of IAB node release was discussed at RAN3#103bis. The proposals raised in this document capture the outcome of the offline discussion in a TP for IAB BL CR to TS 38.401 given in the Annex.

# Discussion

Based on the discussion, the following is proposed:

Proposal 1		The existing NAS Deregistration procedure shall be reused for the IAB node release procedure.

Proposal 2		The existing NGAP UE Context Release procedure shall be reused for releasing the MT context in the RAN.

Proposal 3	Agree that, for the disorderly release case, it should be left to network implementation how to cope with the issue of hanging contexts.

Proposal 4	Agree to the TP for IAB BL CR for TS 38.401, presented in the Annex.

# Conclusion

Based on the above, we propose the following:

Proposal 1		The existing NAS Deregistration procedure shall be reused for the IAB node release procedure.

Proposal 2		The existing NGAP UE Context Release procedure shall be reused for releasing the MT context in the RAN.

Proposal 3	Agree that, for the disorderly release case, it should be left to network implementation how to cope with the issue of hanging contexts.

Proposal 4	Agree to the TP for IAB BL CR for TS 38.401, presented in the Annex.

# Annex: TP to IAB BL CR for TS 38.401

-------------------------------------------Change 1-------------------------------------------

8.9.x 	IAB-node release

An IAB node may depart the network either in an orderly fashion, which implies that both the network and the IAB-node are aware in advance, or in a disorderly fashion (e.g. RLF with failed recovery).

8.9.x.1 	IAB-node orderly release

For orderly release, the IAB-MT is deregistered. The deregistration procedure is the same as the normal UE deregistration procedure. The IAB-donor-CU hands over the UEs or child IAB-nodes currently connected to the IAB-node’s cell(s) to another cell(s), or releases the UEs and may stop accepting incoming handovers or connections to the IAB-node that is about to be released. The IAB-donor-CU may also update/release the backhaul RLC channels in the intermediate hops. At this point, the F1-AP will be release and the corresponding SCTP associations will be removed.

8.9.x.2 	IAB-node disorderly release

For the disorderly release case, how to remove the IAB-node context is up to network implementation.

-------------------------------------------End of changes-------------------------------------------