3GPP TSG-RAN WG3 Meeting #103	R3-192107

# Athens, Greece, 25th, February – 1st March 2019

Agenda Item:	9.3.16

Source:	Ericsson

Title: 	Summary of offline discussion

Document for:	Discussion and Approval

# 1	Way Forward

CB#2 Op1_WF

- Focus on signalling impact over interfaces

- Figure out any left open issues

- Capture agreement

(E///)

RAN3 discussed the signalling aspects for the Solution 1 of Key Issue 1.

The “Dual Connectivity based end to end Redundant User Plane Paths” (the solution 1 of Key Issue 1)  described in TS 23.501, SA2 states how the RSN (Redundancy Sequence Number) is used:

When the NG-RAN contexts are established for the redundant PDU Sessions (i.e., during the establishment of the PDU Sessions or at transitions to CM-CONNECTED state), for one PDU Session the RSN parameter explicitly requests that the user plane goes via the Master RAN, and for the other PDU Session the RSN parameter explicitly requests that the user plane goes via the Secondary RAN using dual connectivity. This request is made by indicating the RSN to the RAN node on a per PDU Session granularity. Based on the RSN, the Master RAN sets up dual connectivity as defined in TS 37.340 [31] so that the sessions have end to end redundant paths. When there is at least one PDU Session with RSN indicating the user plane to go via the Master RAN and at least one PDU Session with RSN indicating the user plane to go via the Secondary RAN node, this indicates to RAN that CN is requesting dual connectivity to be set up and the user plane shall be handled as indicated by the RSN parameter.

RAN3 prefers the 5GC to inform the NG-RAN for which PDU Sessions redundant radio network transport and radio resources shall be setup. NGAP signalling does not need to distinguish MN and SN.

Such indication is sufficient for the NG-RAN to understand to configure for one PDU session  the MN terminated bearers with MCG resources only and for the other PDU session  the SN terminated bearers with SCG resources only.

There is no need for the 5GC to mandate which of the PDU sessions are realised by the MN and which by the SN

An LS is proposed to send to SA2 to kindly feedback the above to SA2.