3GPP TSG RAN WG3#104 	                                                                               R3-192267

Reno, USA, 13th – 17th  May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | pCR | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TR38.821 Clarification of NTN Architecture Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current baseline TR38.821, for gNB on board case in sub-section 5.2.1.1, it has been described that: “The gNB on board different satellites may be connected to the same 5GCN on the ground.” and “If the satellite hosts more than one gNB, the same SRI will transport all the corresponding NG interface instances.”. Similarly, for gNB-DU on board case in sub-section 5.3.2.1, it has been also described that:” DU on board different satellites may be connected to the same CU on ground. and “If the satellite hosts more than one DU, the same SRI will transport all the corresponding F1 interface instances.” However, the counterpart statements for transpararent satellite case is still missing, hence different NTN architecture options were not well described with the same level of details. Since it has been clear that the same SRI will transport all the corresponding NG/F1 interface instances, but the counterpart transport support with ISL is still missing, i.e. whether the same ISL will transport all the corresponding NG/F1 interface instances as well? We assume ISL has the same transport characteristics as SRI, hence the same ISL will transport all the corresponding NG/F1 interface instances. Note: The SRI/ISL transport for Xn interface instances is to be discussed separately in R3-19xxxx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add up two statements in sub-section 5.1.1 that “Different transparent satellites may be connected to the same gNB on the ground.” and “If the transparent satellite hosts more than one RF repeater, the same SRI will transport all the corresponding signals for NR Uu interface.” To clairfy that the same ISL will transport all the corresponding NG/F1 interface instances, if configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion or incompleteness exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.2.1.1, 5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

### 5.1.1	Overview

The satellite payload implements frequency conversion and a Radio Frequency amplifier in both up link and down link direction. It corresponds to an analogue RF repeater.

Hence the satellite repeats the NR-Uu radio interface from the feeder link (between the NTN gateway and the satellite) to the service link (between the satellite and the UE) and vice versa.

The Satellite Radio Interface (SRI) on the feeder link is the NR-Uu. In other words, the satellite does not terminate NR-Uu.

The NTN GW supports all necessary functions to forward the signal of NR-Uu interface.

Different transparent satellites may be connected to the same gNB on the ground.

If the transparent satellite hosts more than one RF repeater, the same SRI will transport all the corresponding signals for NR Uu interface.

Figure 5.2-1: Networking-RAN architecture with transparent satellite

Note: Whilst several gNBs may access a single satellite payload, the description has been simplified to a unique gNB accessing the satellite payload, without loss of generality.

*** Next change, ommited text not changed ***

5.2.1.1	Overview

The NG-RAN logical architecture as described in TS 38.401 is used as baseline for NTN scenarios.

The satellite payload implements regeneration of the signals received from Earth.

- NR-Uu radio interface on the service link between the UE and the satellite

- Satellite Radio Interface (SRI) on the feeder link between the NTN gateway and the satellite.

SRI (Satellite Radio Interface) is a transport link between NTN GW and satellite.

Figure 5.3-1: Regenerative satellite without ISL, gNB processed payload

Note: The satellite may embark additional traffic routing functions that are out of RAN scope. (FFS)

The satellite payload also provides Inter-Satellite Links (ISL) between satellites

ISL (Inter-Satellite Links) is a transport link between satellites. ISL may be a radio interface or an optical interface that may be 3GPP or non 3GPP defined but this is out of the study item scope.

The NTN GW is a Transport Network Layer node, and supports all necessary transport protocols.

Figure 5.3-2: Regenerative satellite with ISL, gNB processed payload (FFS)

The figure above illustrates that UE served by a gNB on board a satellite could access the 5GCN via ISL.

The gNB on board different satellites may be connected to the same 5GCN on the ground.

If the satellite hosts more than one gNB, the same SRI and ISL will transport all the corresponding NG interface instances.

*** Next change, ommited text not changed ***

5.3.2.1	Overview

The NG-RAN logical architecture with CU/DU split as described in TS 38.401 is used as baseline for NTN scenarios.

The satellite payload implements regeneration of the signals received from Earth.

- NR-Uu radio interface on the service link between the satellite and the UE

- Satellite Radio Interface (SRI) on the feeder link between the NTN gateway and the satellite. The SRI transports the F1 protocol.

The satellite payload may provide inter-satellite links between satellites.

SRI (Satellite Radio Interface) are transport links; the logical interface F1 that they transport are 3GPP-specified.

The NTN GW is a Transport Network Layer node, and supports all necessary transport protocols.

DU on board different satellites may be connected to the same CU on ground.

If the satellite hosts more than one DU, the same SRI and ISL will transport all the corresponding F1 interface instances.

Figure 5.3-6: NG-RAN with a regenerative satellite based on gNB-DU

*** End change ***