3GPP TSG-RAN WG3 Meeting #104	R3-192441

Reno, NV, USA, May 13th – 17th 2019

Agenda Item:	   16.2

Source:	   Ericsson

Title:	(TP for NR-CLI BL CR for TS 38.423): CLI info in Neighbour Information NR

Document for:	   Agreement

# Introduction

At the RAN3#103bis meeting, the inclusion of newly-defined Intended TDD DL-UL Configuration NR IE into the Neighbor Information NR IE was denoted as FFS.

In this paper we propose to remove the Intended TDD DL-UL Configuration NR IE form Neighbor Information NR, since the information therein is irrelevant for the recipient.

Proposal: Agree to the TP for CLI BL CR to TS 38.423, presented in the Annex.

# Annex: TP for NR_CLI BL CR to TS 38.423

-------------------------------------------Change 1-------------------------------------------

#### 9.2.2.13	Neighbour Information NR

This IE contains cell configuration information of NR cells that a neighbour NG-RAN node may need to properly operate its own served cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Information NR |  | 1 .. <maxnoofNeighbours> |  |  |
| >NRPCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >NR CGI | M |  | 9.2.2.7 |  |
| >TAC | M |  | 9.2.2.5 | Tracking Area Code |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  |
| >CHOICE NR-Mode-Info | M |  |  |  |
| >>FDD |  |  |  |  |
| >>>FDD Info |  | 1 |  |  |
| >>>>UL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  |
| >>>>DL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  |
| >>TDD |  |  |  |  |
| >>>TDD Info |  | 1 |  |  |
| >>>>NR FreqInfo | M |  | NR ARFCN Frequency Info 9.2.2.19 |  |
| >Connectivity Support | M |  | 9.2.2.28 |  |
| >Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the neighbour cell, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |

------------------------------------------End of Changes-------------------------------------------