3GPP TSG-RAN3 Meeting #104	R3-192665

# Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1328 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of CHO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE CHO is agreed in RAN2. The X2AP messages to enable LTE CHO should introduced in 36.423. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of conditional handover. - Revise the HO preparation procedure and messages to cover CHO case; - Revise HO Cancel message to cover source intiated CHO cancel case; - Add HO Cancel Requried procedure and messages to cover target initiated CHO cancel case; - Add Source initiated CHO modification preparation procedure and messages; - Add Target initiated CHO modification procedure and messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conditional handover is not supported in LTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.y, 8.2.z, 8.2.k, 9.1.1.1, 9.1.1.2, 9.1.1.6, 9.1.1.x1, 9.1.1.x2, 9.1.1.x3, 9.1.1.x4, 9.1.1.x5, 9.1.1.x6, 9.1.1.x7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------Change Start-----------------------------------------------------

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

<No change part is omitted>

If the CHO Indicator IE is included in the HANDOVER REQUEST message, the target eNB shall take this handover as conditional handover.

If the CHO Indicator IE is included in the HANDOVER REQUEST message, the target eNB shall take the target cell included in CHO Cell To Be Added List as candidate target cell for the conditional handover. If the target eNB can admit the UE in the candidate target cell, the target eNB shall include the target cell in the CHO Cell Admitted To Be Added List IE.

-----------------------------------------------------Next Change-----------------------------------------------------

8.2.y	Handover Cancel Required

8.2.y.1	General

The Handover Cancel Required procedure is used to enable a target eNB to cancel an ongoing conditional handover preparation or an already prepared conditional handover.

The procedure uses UE-associated signalling.

8.2.y.2	Successful Operation

Figure 8.2.4.2-1: Handover Cancel, successful operation

The target eNB initiates the procedure by sending the HANDOVER CANCEL REQUIRED message to the source eNB. The target eNB shall indicate the reason for cancelling the handover by means of an appropriate cause value.

8.2.y.3	Unsuccessful Operation

Not applicable.

-----------------------------------------------------Next Change-----------------------------------------------------

8.2.z	Source initiated CHO Modification Preparation

8.2.z.1	General

This procedure is used to enable a source eNB to request a target to modify the UE CHO configuration during a conditional handover.

The procedure uses UE-associated signalling.

8.2.z.2	Successful Operation

Figure 8.2.z.2-1: Source eNB initiated CHO Modification Preparation, successful operation

The source eNB initiates the procedure by sending the CHO MODIFICATION REQUEST message to the target eNB.

The CHO MODIFICATION REQUEST message may contain CHO Cell To Be Added List IE, CHO Cell To Be Modified List IE, CHO Cell To Be Released List IE.

8.2.z.3	Unsuccessful Operation

Figure 8.2.z.3-1: Source eNB initiated CHO Modification Preparation, unsuccessful operation

If the target eNB does not not admit any modification requested by the source eNB, or a failure occurs during the source eNB initiated CHO Modification Preparation, the target eNB shall send the CHO MODIFICATION REQUEST FAILURE message to the source eNB. The message shall contain the Cause IE with an appropriate value.

-----------------------------------------------------Next Change-----------------------------------------------------

8.2.k	Target initiated CHO Modification

8.2.k.1	General

This procedure is used by the target eNB to modify the CHO configuration.

The procedure uses UE-associated signalling.

8.2.k.2	Successful Operation

Figure 8.2.k.2-1: Target initiated CHO Modification, successful operation.

The target eNB initiates the procedure by sending the CHO MODIFICATION REQUIRED message to the source eNB.

The CHO MODIFICATION REQUIRED message may contain CHO Cell To Be Modified List IE, CHO Cell To Be Released List IE.

8.2.k.3	Unsuccessful Operation

Figure 8.2.k.3-1: Target initiated CHO Modification, unsuccessful operation.

In case the request modification cannot be performed successfully the Source eNB shall respond with the CHO MODIFICATION REFUSE message to the target eNB with an appropriate cause value in the Cause IE.

-----------------------------------------------------Next Change-----------------------------------------------------

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover or conditional handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – | – |
| >UE Security Capabilities | M |  | 9.2.29 |  | – | – |
| >AS Security Information | M |  | 9.2.30 |  | – | – |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – | – |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – | – |
| >E-RABs To Be Setup List |  | 1 |  |  | – | – |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the RRC HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – | – |
| >Handover Restriction List | O |  | 9.2.3 |  | – | – |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – | – |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | Ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  |  |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB |  |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB |  |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  |  |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  |  |  |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  |  |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – | – |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| CHO Indicator | O |  |  | Indicates this HO Request is for a conditional handover. |  |  |
| CHO Cell To Be Added List | O |  |  |  |  |  |
| >CHO Cell To Be Added Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9]. | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target eNB to inform the source eNB about the prepared resources at the target.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| E-RABs Admitted List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – | – |
| >>DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – | – |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| WT UE Context Kept Indicator | O |  | UE Context Kept Indicator 9.2.85 | Indicates that the WT has acknowledged to keep the UE context | YES | ignore |
| CHO Cell Admitted To Be Added List | O |  |  |  |  |  |
| >CHO Admitted To Be Added Item |  | 1 .. <maxnoofCHOcells> | ECGI 9.2.14 |  | YES | reject |
| >>Target Cell ID | M |  |  |  |  |  |
| >> Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| CHO Cell Not Admitted List | O |  |  |  |  |  |
| >CHO Cell Not Admitted Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>Cause | M |  | 9.2.6 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

-----------------------------------------------------Next Change-----------------------------------------------------

9.1.1.6	HANDOVER CANCEL

This message is sent by the source eNB to the target eNB to cancel an ongoing handover or conditional handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | ignore |

-----------------------------------------------------Next Change-----------------------------------------------------

9.1.1.x1	CHO CANCEL REQUIRED

This message is sent by the target eNB to the source eNB to cancel an ongoing conditional handover.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |

-----------------------------------------------------Next Change-----------------------------------------------------

9.1.1.x2	CHO MODIFICATION REQUEST

This message is sent by the source eNB to the target eNB to request the preparation to modify target cell resources for a specific UE, e.g. change the target cell(s).

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB. | YES | reject |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB. | YES | reject |
| CHO Cell To Be Added List | O |  |  |  |  |  |
| >CHO Cell To Be added Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9]. | – | – |
| CHO Cell To Be Modified List | O |  |  |  |  |  |
| >CHO Cell To Be Modifed Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9]. | – | – |
| CHO Cell To Be Released List | O |  |  |  |  |  |
| >CHO Cell To Be Release Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>Cause | M |  | 9.2.6 |  | YES | ignore |

9.1.1.x3	CHO MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the target eNB to confirm the source eNB’s request to modify the target eNB resources for a specific UE.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| CHO Cell Admitted To Be Added List | O |  |  |  |  |  |
| >CHO Cell Admitted To Be added Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >> Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| CHO Cell Admitted To Be Modified List | O |  |  |  |  |  |
| >CHO Cell Admitted To Be Modifed Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >> Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| CHO Cell Admitted To Be Released List | O |  |  |  |  |  |
| >CHO Cell To Be Released Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| CHO Cell Not Admitted List | O |  |  |  |  |  |
| >CHO Cell Not Admitted Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>Cause | M |  | 9.2.6 |  | YES | ignore |

9.1.1.x4	CHO MODIFICATION REQUEST REJECT

This message is sent by the target eNB to inform the source eNB that the source eNB initiated CHO Modification Preparation has failed.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |

9.1.1.x5	CHO MODIFICATION REQUIRED

This message is sent by the target eNB to the source eNB to request the modification of target eNB resources for a specific UE, e.g. release target cell(s).

Direction: en-gNB  MeNB.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| CHO Cell To Be Modified List | O |  |  |  |  |  |
| >CHO Cell To Be Modifed Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9]. | – | – |
| CHO Cell To Be Released List | O |  |  |  |  |  |
| >CHO Cell To Be Release Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>Cause | M |  | 9.2.6 |  | YES | ignore |

9.1.1.x6	CHO MODIFICATION CONFIRM

This message is sent by the source eNB to inform the target eNB about the successful modification.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| CHO Cell Admitted To Be Modified List | O |  |  |  |  |  |
| >CHO Cell Admitted To Be Modifed Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| CHO Cell Admitted To Be Released List | O |  |  |  |  |  |
| >CHO Cell To Be Released Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| CHO Cell Not Admitted List | O |  |  |  |  |  |
| >CHO Cell Not Admitted Item |  | 1 .. <maxnoofCHOcells> |  |  |  |  |
| >>Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| >>Cause | M |  | 9.2.6 |  | YES | ignore |

9.1.1.x7	CHO MODIFICATION REFUSE

This message is sent by the source eNB to inform the target eNB that the source eNB initiated CHO Modification has failed.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |