3GPP TSG-RAN WG3 Meeting #104	R3-192753

Reno, NV, USA, May 13th – 17th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0288 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1AP Signalling for Remote Interference Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM |  |  |  |  |  |  | Date: |  | 2019-05-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling Remote Interference Management signaling over F1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the CU-DU Radio Information Transfer and DU-CU Radio Information Transfer Class-2 procedures. Introduced the RIM-specific IEs. This is a backwards-compatible CR. The corresponding discussion is presented in R3-192751. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote Interference Management not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.1, 8.x (new), 9.2.x.1 (new), 9.2.x.2 (new), 9.3.1.x (new), 9.3.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Change 1-------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CP	Control Plane

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IMEISV 	International Mobile station Equipment Identity and Software Version number

NSSAI	Network Slice Selection Assistance Information

RANAC	RAN Area Code

RIM	Remote Interference Management

RRC	Radio Resource Control

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

-------------------------------------------Change 2-------------------------------------------

## 8.1	List of F1AP Elementary procedures

>>>>>>>>>>>>>>Skipped text<<<<<<<<<<<<<<

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION |

-------------------------------------------Change 3-------------------------------------------

## 8.x	Radio Information Transfer procedures

### 8.x.1	DU-CU Radio Information Transfer

#### 8.4.1.1	General

The purpose of the DU-CU Radio Information Transfer procedure is to transfer radio-related information from the gNB-DU to the gNB-CU. The procedure uses non-UE-associated signalling.

#### 8.4.1.2	Successful operation

Figure 8.4.1.2-1: DU-CU Radio Information Transfer procedure.

If the RIM Backhaul Message IE is included in the DU-CU RADIO INFORMATION MESSAGE, the gNB-CU should merge this message with the messages containing RIM Backhaul Message IEs received from other gNB-DUs and forward the merged message towards the gNBs, gNB-CUs or gNB-DUs affiliated with the Target RIM Set ID IE value inside the RIM Backhaul Message IE.

#### 8.x.1.3	Abnormal Conditions

Not applicable.

### 8.x.2	CU-DU Radio Information Transfer

#### 8.x.2.1	General

The purpose of the CU-DU Radio Information Transfer procedure is to transfer radio-related information from the gNB-CU to the gNB-DU. The procedure uses non-UE-associated signalling.

#### 8.x.2.2	Successful operation

Figure 8.4.2.2-1: CU-DU Radio Information Transfer procedure.

If the RIM Backhaul Message IE is included in the CU-DU RADIO INFORMATION MESSAGE, the gNB-DU shall, if supported, take the information therein into account for Remote Interference Management. The gNB-CU shall send this message to all the gNB-DUs affiliated with the Target RIM Set ID value inside the RIM Backhaul Message IE.

#### 8.x.2.3	Abnormal Conditions

Not applicable.

-------------------------------------------Change 4-------------------------------------------

#### 9.2.x.1	DU-CU RADIO INFORMATION

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| CHOICE Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  | - |  |
| >>RIM Backhaul Message | M |  | 9.3.1.x |  | YES | ignore |

-------------------------------------------Change 5-------------------------------------------

#### 9.2.x.1	CU-DU RADIO INFORMATION

This message is sent by a gNB-CU to a gNB-DU, to convey radio-related information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject ignore |
| CHOICE Radio Information Type | M |  |  |  | YES | reject ignore |
| >RIM |  |  |  |  | - |  |
| >>RIM Backhaul Message | M |  | 9.3.1.x |  | YES | reject ignore |

-------------------------------------------Change 6-------------------------------------------

#### 9.3.1.x	RIM Backhaul Message

This IE conveys the Remote Interference Management message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source RIM Set ID | M |  | 9.3.1.y | ID of a RIM set. | YES | ignore |
| Target RIM Set ID | M |  | 9.3.1.y | ID of a RIM set. | YES | ignore |
| List of Cells |  | 1 |  |  |  |  |
| > List of Cells Item |  | 1..< maxCellingNBDU > |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| RIM Info | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational content of Remote Interference Management Backhaul Message. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

-------------------------------------------Change 7-------------------------------------------

#### 9.3.1.y	RIM Set ID

This IE defines the Remote Interference Management Set ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Set ID | M |  | BIT STRING (SIZE(22)) | The Remote Interference Management Set ID. |

-------------------------------------------End of changes ------------------------------------------