3GPP TSG-RAN3 Meeting #104	R3-192916

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0086 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session split for E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Last RAN3#103 meeting agreed the stage 2 and TS 38.423 CR to support the in order delivery for MR-DC in [R3-190953] and [R3-190336] respectively. Specifically, the UL PDU session tunnel is setup if the in-order delivery is requried in the RAN triggered PDU split. The same principle should be applied to E1 interface as well. When the CU-UP as the target NG-RAN node, should perform the following steps in Bearer Context Setup or Modification procedure - It receives the UL forwarding request; - It allocates the UL PDU session tunnel and notify the CU-CP. For the first step, the current Semantics description of QoS Flows forwarded on the forwarding tunnel IE within the Data Forwarding Informaton Request IE only includes the DL PDU session forwarding tunnel, which should be corrected. The second step is already supported in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To correct the Semantics description of QoS Flows forwarded on the forwarding tunnel IE within the Data Forwarding Informaton Request IE to include the UL PDU session forwarding tunnel in addtion to the DL PDU session forwarding tunnel. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the bearer context setup or bearer context modification function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL PDU session tunnel is not setup between the source node and the target CU-UP. The in order delivery can not ensured in the RAN triggered PDU split |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2,8.3.2.2,9.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

<Unchanged Text Omitted>

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message. If the Data Forwarding Request IE is set as UL or both in the PDU Session Data Forwarding Information Request IE in the BEARER CONTEXT SETUP REQUEST message for forwarding of UL data when in-order delivery of the QoS flows is needed, the gNB-CU-UP may include the UL Data Forwarding IE in the PDU Session Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message to indicate that it accepts the proposed forwarding.

Next Change

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

<Unchanged Text Omitted>

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message. If the Data Forwarding Request IE is set as UL or both in the PDU Session Data Forwarding Information Request IE in the BEARER CONTEXT MODIFICATION REQUEST message for forwarding of UL data when in-order delivery of the QoS flows is needed, the gNB-CU-UP may include the UL Data Forwarding IE in the PDU Session Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message to indicate that it accepts the proposed forwarding.

Next Change

9.3.2	Transport Network Layer Related IEs

<Unchanged Text Omitted>

9.3.2.5	Data Forwarding Information Request

This IE offers the possibility for the gNB-CU-CP to request data forwarding addresses to the gNB-CU-UP. It also offers the possibility for the gNB-CU-CP to provide a list of QoS flows subject to PDU session level or DRB level data forwarding to the gNB to which DRBs or QoS flows have been offloaded.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding Request | M |  | ENUMERATED (UL, DL, both, …) |  |
| QoS Flows forwarded on the forwarding tunnel(s) | O |  | QoS Flow Mapping List 9.3.1.59 | This IE contains information for which QoS flows forwarded data packets are sent on: - either the PDU Session forwarding tunnel (UL and DL) - or the DRB forwarding tunnel (UL and DL). |

End of change